|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | Q.751.4 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | (05/98) |

SERIES Q: SWITCHING AND SIGNALLING

Specifications of Signalling System No. 7 – Signalling System No. 7 management

NETWORK ELEMENT INFORMATION MODEL FOR SCCP ACCOUNTING AND ACCOUNTING VERIFICATION

FOREWORD

CAUTION !

PREPUBLISHED  RECOMMENDATION

This text is a non-edited version of a recently approved Recommendation. It shall be replaced by the final version after edition. Consequently, some discrepancies may appear between this text and its final edited form

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU   1998

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation Q.751.4

Network Element Information Model For SCCP
Accounting and Accounting Verification

Summary

Measurements for SCCP Accounting and Accounting Verification have been defined in Q.752. The network element information model - based on X.742 and Q.751.2 - for these measurements is contained in this Recommendation.

Keywords

SCCP, Accounting, Verification, Measurements, OMAP, object model, network element information model, TMN

CONTENTS

Page

1	Scope		3

2	References		3

3	Terms and definitions		4

4	Abbreviations		5

5	Conventions		5

6	Informal description of managed object classes		6

6.1	Reused Managed Objects		6

6.2	Relation Notification/LogRecord/File		6

6.3	Diagrams		6

6.3.1	Inheritance diagram		6

6.3.2	ER-Diagram for SCCP Accounting and Verification		7

6.4	Textual description		9

6.5	Tables		10

7	Formal definitions		15

7.1	Formal definitions common for SS7 accounting		16

7.2	Formal definitions for SCCP accounting		16

7.2.1	Managed object class definitions		16

7.2.2	Package definitions		17

7.2.3	Attribute definitions		19

7.2.4	Name binding definitions		20

7.2.5	Notification definitions		21

7.2.6	Parameter definitions		22

7.2.7	ASN.1 definitions		22

Figures

Q.751.4/Figure 1 - Inheritance diagram		7

Q.751.4/Figure 2 - ERD - Diagram for SCCP accounting and verification		8

Tables

Q.751.4/Table 1 - Reuse of Q.751.1, Q.751.2 and Q.751.3		6

Q.751.4/Table 2 - sccpAccount		11

Q.751.4/Table 3 - terminatingAccountClassForAccounting		12

Q.751.4/Table 4 - terminatingAccountClassForVerification		13

Q.751.4/Table 5 - verificationRule		14

Q.751.4/Table 6 - verificationTranslator		15

### 1	Scope

This Recommendation contains the network element information model which is necessary to manage network elements for accounting and accounting verification of SS7 SCCP traffic according to ITU-T Recommendation Q.752. The model is based on the models defined in ITU-T Recommendations Q.751.2 and X.742. SCCP Accounting options which are marked "f.f.s." in Q.752 are not included.

### 2	References

The following ITU-T Recommendations, and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

ITU-T Recommendation A.1500, Elaboration and presentation of texts for Recommendations of the ITU Telecommunications Standardization Sector, 1993.

ITU-T Recommendation I.751 Asynchronous Transfer Mode (ATM) Management of the Network Element View, 1995.

ITU-T Recommendation M.3100, Generic Network Information Model, 1992.

ITU-T Recommendation Q.750, Overview of Signalling System No. 7 Management, 1993.

ITU-T Recommendation Q.751.1, Network Element Information Model for the Message Transfer Part (MTP), 1995.

ITU-T Recommendation Q.751.2, Network Element Information Model for the Signalling Connection Control Part (SCCP), 1996.

ITU-T Recommendation Q.751.3, Network Element Information Model for MTP Accounting, 1997.

ITU-T Recommendation X.208, Specification of Abstract Syntax Notation One (ASN.1), 1988.

ITU-T Recommendation X.680, Information technology - Open System Interconnection - Abstract Syntax Notation One (ASN.1): Specification of basic notation, 1994.

ITU-T Recommendation X.711, Common Management Information Protocol Specification for CCITT applications, 1991.

ITU-T Recommendation X.720, Management Information Model, 1992.

ITU-T Recommendation X.721, Definition of management information, 1992.

ITU-T Recommendation X.722, Guidelines for the Definition of Managed Objects, 1992.

Amendment 1 to ITU-T Recommendation X.722: SET BY CREATE And Component Registration, 1995.

ITU-T Recommendation X.723, Structure of Management Information: Generic Management Information, 1992.

ITU-T Recommendation X.731, State Management Function, 1992.

ITU-T Recommendation X.733, Alarm Reporting Function, 1992.

ITU-T Recommendation X.742, Usage Metering Function For Accounting Purposes, 1995.

# 3	Terms and definitions

For the purpose of this Recommendation, the following definitions apply:

This Recommendation makes use of the following terms defined in ITU-T Recommendation M.3010:

a)	performance management;

b)	configuration management;

c)	fault management;

d)	telecommunications management network (TMN).

This Recommendation makes use of the following terms defined in ITU-T Recommendation X.700:

a)	object instance.

This Recommendation makes use of the following terms defined in ITU-T Recommendation X.701

a)	managed object class;

b)	management information;

c)	notification.

This Recommendation makes use of the following terms defined in ITU-T Recommendation X.710:

a)	attribute.

This Recommendation makes use of the following terms defined in ITU-T Recommendation X.720:

a)	inheritance;

b)	name binding;

c)	package;

d)	parameter;

e)	action;

f)	actual class;

g)	attribute group;

h)	behaviour;

i)	conditional package;

j)	instantiation;

k)	superclass;

This Recommendation | International Standard makes use of the following terms defined in CCITT Recommendation X.701 | ISO/IEC 10040:

a)	managed object class;

b)	notification.

# 4	Abbreviations

For the purpose of this Recommendation, the following definitions apply:

ASN.1		Abstract Syntax Notation 1

ERD		Entity Relationship Diagram

GDMO		Guidelines for the Definition of Managed Objects

max		Maximum

MO		Managed Object

MOC		Managed Object Class

MSU 		Message Signal Unit

MTP		Message Transfer Part

NE		Network Element

OMAP		Operations, Maintenance and Administration Part

# 5	Conventions

ITU-T Recommendation A.1500 (Elaboration and presentation of texts for Recommendations of the ITU-T) is used.

The Guidelines for the Definition of Managed Objects (GDMO), defined in Recommendation X.722, are used. In case of differences between the formal part (Clause 8) and the informal parts of this Recommendation the formal part is to be regarded as leading.

If there are inconsistencies between the informal description, the formal definitions or conformance statements proformas, the formal definitions shall prevail.

Throughout this Recommendation the wording "The managed object class x ......" refers to a particular managed object class while the wording "An x ....." refers to an instance of the managed object class "x".

Modelling of redundancy is avoided (e.g. relationships between managed objects are described in one MOC only, information which is obtainable via referenced instances of other information models are not repeated here). However, for some implementations, it may be useful or necessary to add some additional information to some managed object classes.

# 6	Informal description of managed object classes

This clause gives informal descriptions of the object model for SS7 SCCP accounting and accounting verification.

## 6.1	Reused Managed Objects

The following Table 1 gives an overview of all the Q.751.1, Q.751.2 and Q.751.3 object classes which are referenced by object classes of this information model.

Table 1/Q.751.4

Reuse of Q.751.1, Q.751.2 and Q.751.3

| referenced object class | Q.751.1/ Q.751.2/ Q.751.3 |
| --- | --- |
| managedSwitchingElement | Q.751.1 |
| sccp | Q.751.2 |
| scrc | Q.751.2 |
| sccpLinkage | Q.751.2 |
| gtTranslator | Q.751.2 |
| gtRule | Q.751.2 |
| ss7AccountingAndVerificationControl | Q.751.3 |

## 6.2	Relation Notification/LogRecord/File

See ITU-T Recommendation Q.751.3

## 6.3	Diagrams

### 6.3.1	Inheritance diagram

Figure /Q.751.4

Inheritance diagram

### 6.3.2	ER-Diagram for SCCP Accounting and Verification

#### Explanation of arrows in the ERD

Containment (full line):

Double arrows at the subordinate object class signify that several instances of it can be contained in one instance of the superior object class.

Relation (dashed line):

A single arrow at the "targeted" object means that the "pointing" object references exactly one instance of the "target" object.

Double arrows at the "targeted" object mean that the "pointing" object can reference several instances of the "target" object. A single arrow at the "pointing" object means that the "targeted" object is referenced by exactly one instance of the "pointing object.

Double arrows at the "pointing" object mean that the "targeted" object can be referenced by several instances of the "pointing object.

Figure 2/Q.751.4

ERD - Diagram for SCCP accounting and verification

Remark

SCCP Accounting options (selection based on SSN or SCCP Protocol Class) which are marked "f.f.s." in Q.752 are not included. Extension for these options could be modelled as follows: Object classes defining the registration items must be defined, if it is not already existing in Q.751.2. The attributes selectionGroupSetForAccounting and selectionGroupSetForVerification then point optionally to the corresponding object class. The attribute type for these attributes has to be extended for an additional optional pointer to the corresponding object class.

## 6.4	Textual description

The information model presented in Figure 1 contains 4 SCCP Accounting & Accounting Verification specific objects: sccpAccount, verificationTranslator, verificationRule, terminatingAccountClassForAccounting and terminatingAccountClassForVerification (ss7AccountingAndVerificationControl is common for MTP and SCCP Accounting & Accounting Verification and is defined in ITU-T Recommendation Q.751.3).

An sccpAccount represents a set of remote sccp nodes (represented by sccpLinkages) belonging to one operator for which accounting/accounting verification may be done collectively. For each sccpAccount, the accountable traffic may be recorded into different counters according to certain selection criteria that are different for accounting/accounting verification. The counters are represented by a set of references to terminatingAccountClass(ForAccounting/Verification) instances.

A terminatingAccountClassForAccounting contained in scrc groups a number of gtRules for which the same tariff applies for accounting purposes. A reference to an instance of this class in sccpAccount represents an accounting counter with a specific tariff valid for the affected operator.

–	This containment in scrc means that the grouping that applies for accounting is identical for all remote operators SCCP Accounting bills are sent to. Prices associated with the accountclasses are however not restricted by this rule.

–	The fact that the groupings apply for all sccpAccounts, also means that the gtRules belong to only one terminatingAccountClassForAccounting. Otherwise, a unique selection of a tariff/counter would not be possible.

A terminatingAccountClassForVerification contained in an sccpAccount groups a number of verificationRules for which the same tariff applies for accounting verification purposes. A reference to an instance of this class in sccpAccount represents a verification counter with a specific tariff valid for the affected operator.

–	This containment in sccpAccount means that the grouping only applies for the operator specified by the sccpAccount. The verificationRules belonging to this tariff (terminatingAccountClassForVerification) shall also belong to this operator, i.e. the superior verificationTranslator shall be contained in the same sccpAccount.

–	Within the scope of an sccpAccount, the verificationTranslators also belong to only one terminatingAccountClassForVerification.

A gtTranslator is used during the global title translation process for routing purposes. The verificationTranslator has the same purpose during the global title analysis process for accounting verification purposes. For each sccpAccount, a set of verificationTranslator are to be defined.

A gtRule is used during the global title translation process for routing purposes. The verificationRule has the same purpose during the global title analysis process for accounting verification purposes. For each sccpAccount, a set of verificationrules are to be defined. These rules are assigned to a terminatingAccountClassForVerification representing a verification counter for the affected operator.

## 6.5	Tables

In these tables, (I), (M) and (C) are used with the following meaning:

–	(I)		This element is inherited from a superclass

–	(M)		This element is mandatory

–	(Cn)	Conditional, n specifies the number of the condition; explanation of it below the tables

–	(O)		Optional (condition "if the instance supports it" or similar)

This signification is only done at the package level.

The possible operations on objects and attributes are specified by:

–	(Cr)	CREATE

–	(WrOI)	WITH REFERENCE OBJECT INSTANCE

–	(Del)	DELETE

–	(oinco)	ONLY-IF-NO-CONTAINED-OBJECTS

–	(G)		GET

–	(SBC)	SET BY CREATE

–	(R)		REPLACE

–	(A-Rm)	ADD-REMOVE

–	(d)		DEFAULT VALUE is defined for the attribute

–	(dr)		DEFAULT VALUE DERIVATION RULE is defined for the attributes

### 6.5.1	Managed Objects Classes Common for SS7 Accounting

For the explanations of the object class and the attributes defined in this Recommendation see their behaviour descriptions in the formal definition section.

For objectClass ss7AccountingAndVerificationControl see ITU-T Recommendation Q.751.3.

## 6.5.2	Managed Objects Classes for SCCP Accounting

#### 6.5.2.1	sccpAccount

Table 2/Q.751.4

sccpAccount

| sccpAccount (Cr, Del oinco) |  |  |  |  |
| --- | --- | --- | --- | --- |
| attributes |  | notifications |  | actions |
| sccpAccountPackage (M) |  |  |  |  |
| sccpAccountId (G, SBC) |  |  |  |  |
| sccpLinkageSet (G, SBC, A-Rm) |  |  |  |  |
| operatorName (G, SBC) |  |  |  |  |
| selectionGroupSetForAccounting (G, R, A-Rm, i) |  |  |  |  |
| selectionGroupSetForVerification (G, R, A-Rm, i) |  |  |  |  |
| "ITU-T Rec. M.3100 (1995)":objectManagementNotificationsPackage (M) |  |  |  |  |
|  |  | "ITU-T Rec. X.721 (1992)":attributeValueChange |  |  |
|  |  | "ITU-T Rec. X.721 (1992)":objectCreation |  |  |
|  |  | "ITU-T Rec. X.721 (1992)":objectDeletion |  |  |
| "ITU-T Rec. Q.751.3 (1997)":controlPointerPackage (O) |  |  |  |  |
| "ITU-T Rec. Q.751.3 (1997)":controlPointer (G, SBC) |  |  |  |  |
| "ITU-T Rec. Q.751.3 (1997)":measurementControlStatusPackage (O) |  |  |  |  |
| "ITU-T Rec. Q.751.3 (1997)": measurementControlStatus (G, SBC) |  |  |  |  |
| sccpAccountingNotificationsPackage (O) |  |  |  |  |
|  | sccpAccounting |  |  |  |
|  | sccpAccountingVerification |  |  |  |
| "ITU-T Rec. X.721 (1992)":topPackage (M,I) |  |  |  |  |
| objectClass (G) |  |  |  |  |
| nameBinding (G) |  |  |  |  |
| "ITU-T Rec. X.721 (1992)":packagesPackage (C1,I) |  |  |  |  |
| packages (G) |  |  |  |  |
| "ITU-T Rec. X.721 (1992)":allomorphicPackage (C2,I) |  |  |  |  |
| allomorphs (G) |  |  |  |  |

C1: Present if any registered package has been instantiated.

C2: Present if allomorphism is supported.

#### 6.5.2.2	sccpAccountingLogRecord

This non-instantiable object class is defined to formally supply the possibility to write the sccpAccounting and/or sccpAccountingVerification notifications into a log.

#### 6.5.2.3	terminatingAccountClassForAccounting

Table 3/Q.751.4

terminatingAccountClassForAccounting

| terminatingAccountClassForAccounting (Cr WrOI, Del) |  |  |
| --- | --- | --- |
| attributes | notifications | actions |
| terminatingAccountClassForAccountingPackage (M) |  |  |
| terminatingAccountClassForAccountingId (G, SBC) |  |  |
| ruleSet (G, SBC, A-Rm) |  |  |
| "ITU-T Rec. M.3100 (1995)":objectManagementNotificationsPackage (M) |  |  |
|  | "ITU-T Rec. X.721 (1992)":attributeValueChange |  |
|  | "ITU-T Rec. X.721 (1992)":objectCreation |  |
|  | "ITU-T Rec. X.721 (1992)":objectDeletion |  |
| "ITU-T Rec. X.721 (1992)":topPackage (M,I) |  |  |
| objectClass (G) |  |  |
| nameBinding (G) |  |  |
| "ITU-T Rec. X.721 (1992)":packagesPackage (C1,I) |  |  |
| packages (G) |  |  |
| "ITU-T Rec. X.721 (1992)":allomorphicPackage (C2,I) |  |  |
| allomorphs (G) |  |  |

C1: Present if any registered package has been instantiated.

C2: Present if allomorphism is supported.

#### 6.5.2.4	terminatingAccountClassForVerification

Table 4/Q.751.4

terminatingAccountClassForVerification

| terminatingAccountClassForVerification (Cr WrOI, Del) |  |  |
| --- | --- | --- |
| attributes | notifications | actions |
| terminatingAccountClassForVerificationPackage (M) |  |  |
| terminatingAccountClassForVerificationId (G, SBC) |  |  |
| ruleSet (G, SBC, A-Rm) |  |  |
| "ITU-T Rec. M.3100 (1995)":objectManagementNotificationsPackage (M) |  |  |
|  | "ITU-T Rec. X.721 (1992)":attributeValueChange |  |
|  | "ITU-T Rec. X.721 (1992)":objectCreation |  |
|  | "ITU-T Rec. X.721 (1992)":objectDeletion |  |
| "ITU-T Rec. X.721 (1992)":topPackage (M,I) |  |  |
| objectClass (G) |  |  |
| nameBinding (G) |  |  |
| "ITU-T Rec. X.721 (1992)":packagesPackage (C1,I) |  |  |
| packages (G) |  |  |
| "ITU-T Rec. X.721 (1992)":allomorphicPackage (C2,I) |  |  |
| allomorphs (G) |  |  |

C1: Present if any registered package has been instantiated.

C2: Present if allomorphism is supported.

#### 6.5.2.5	verificationRule

Table 5/Q.751.4

verificationRule

| verificationRule (Cr WrOI, Del) |  |  |
| --- | --- | --- |
| attributes | notifications | actions |
| verificationRulePackage (M) |  |  |
| verificationRuleId (G, SBC) |  |  |
| ITU-T Rec. Q.751.2 (1997):gtAdressInformation (G, R) |  |  |
| ITU-T Rec. Q.751.2 (1997):gtEncodingScheme (G, SBC) |  |  |
| "ITU-T Rec. M.3100 (1995)":objectManagementNotificationsPackage (M) |  |  |
|  | "ITU-T Rec. X.721 (1992)":attributeValueChange |  |
|  | "ITU-T Rec. X.721 (1992)":objectCreation |  |
|  | "ITU-T Rec. X.721 (1992)":objectDeletion |  |
| "ITU-T Rec. X.721 (1992)":topPackage (M,I) |  |  |
| objectClass (G) |  |  |
| nameBinding (G) |  |  |
| "ITU-T Rec. X.721 (1992)":packagesPackage (C1,I) |  |  |
| packages (G) |  |  |
| "ITU-T Rec. X.721 (1992)":allomorphicPackage (C2,I) |  |  |
| allomorphs (G) |  |  |

C1: Present if any registered package has been instantiated.

C2: Present if allomorphism is supported.

#### 6.5.2.6	verificationTranslator

Table 6/Q.751.4

verificationTranslator

| verificationTranslator (Cr WrOI, Del) |  |  |
| --- | --- | --- |
| attributes | notifications | actions |
| verificationTranslatorPackage (M) |  |  |
| verificationTranslator Id (G, SBC) |  |  |
| ITU-T Rec. Q.751.2 (1997):gtIndicator (G, SBC) |  |  |
| gtNatureOfAddressSet (G, R, A-Rm) |  |  |
| gtNumberingPlanSet (G, R, A-Rm) |  |  |
| gtTranslationTypeSet (G, R, A-Rm) |  |  |
| "ITU-T Rec. M.3100 (1995)":objectManagementNotificationsPackage (M) |  |  |
|  | "ITU-T Rec. X.721 (1992)":attributeValueChange |  |
|  | "ITU-T Rec. X.721 (1992)":objectCreation |  |
|  | "ITU-T Rec. X.721 (1992)":objectDeletion |  |
| "ITU-T Rec. X.721 (1992)":topPackage (M,I) |  |  |
| objectClass (G) |  |  |
| nameBinding (G) |  |  |
| "ITU-T Rec. X.721 (1992)":packagesPackage (C1,I) |  |  |
| packages (G) |  |  |
| "ITU-T Rec. X.721 (1992)":allomorphicPackage (C2,I) |  |  |
| allomorphs (G) |  |  |

C1: Present if any registered package has been instantiated.

C2: Present if allomorphism is supported.

# 7	Formal definitions

## 7.1	Formal definitions common for SS7 accounting

These definitions are defined in ITU-T Recommendation Q.751.3 and are reused.

## 7.2	Formal definitions for SCCP accounting

### 7.2.1	Managed object class definitions

sccpAccount MANAGED OBJECT CLASS

DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

"ITU-T Rec. M.3100 (1995)":objectManagementNotificationsPackage,

sccpAccountPackage ;

CONDITIONAL PACKAGES

"ITU-T Rec. Q.751.3 (1997)":controlPointerPackage PRESENT IF "the instance supports it",

"ITU-T Rec. Q.751.3 (1997)":measurementControlStatusPackage PRESENT IF "the instance

supports it",

sccpAccountingNotificationsPackage PRESENT IF "the instance supports it";

REGISTERED AS {sccpAccount-OOi};

sccpAccountingLogRecord MANAGED OBJECT CLASS

DERIVED FROM "ITU-T Rec. X.735 (1991)":eventLogRecord;

CHARACTERIZED BY

sccpAccountingLogRecordPackage;

REGISTERED AS {sccpAccountingLogRecord-OOi};

terminatingAccountClassForAccounting MANAGED OBJECT CLASS

DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

terminatingAccountClassForAccountingPackage,

"ITU-T Rec. M.3100 (1995)":objectManagementNotificationsPackage ;

REGISTERED AS {terminatingAccountClassForAccounting-OOi};

terminatingAccountClassForVerification MANAGED OBJECT CLASS

DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

terminatingAccountClassForVerificationPackage,

"ITU-T Rec. M.3100 (1995)":objectManagementNotificationsPackage ;

REGISTERED AS {terminatingAccountClassForVerification-OOi};

verificationRule MANAGED OBJECT CLASS

DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

verificationRulePackage,

"ITU-T Rec. M.3100 (1995)":objectManagementNotificationsPackage ;

REGISTERED AS {verificationRule-OOi};

verificationTranslator MANAGED OBJECT CLASS

DERIVED FROM "ITU-T Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

verificationTranslatorPackage,

"ITU-T Rec. M.3100 (1995)":objectManagementNotificationsPackage ;

REGISTERED AS {verificationRule-OOi};

### 7.2.2	Package definitions

sccpAccountPackage PACKAGE

BEHAVIOUR sccpAccountPackageBehaviour BEHAVIOUR DEFINED AS

"The sccpAccount managed object allows by means of the attribute sccpLinkageSet to define a set (at least one) of remote nodes for which SCCP accounting/verification can be done collectively, because they are belonging to the same operator. All sccpLinkageSets defined by sccpAccounts shall be disjunct.

The selectionGroupSetForAccounting refers to terminatingAccountClassForAccounting instances contained in scrc, i.e. the accounting is independent of the operator (Other selection items for SCCP accounting are for further study). The selectionGroupSetForVerification refers to terminatingAccountClassForVerification instances contained in the same sccpAccount that the selectionGroupSetForVerification attribute belongs to, i.e. the verification is operator dependent. Each selectionGroup entry shall be unique in these attributes.     For each selectionGroup it is counted separately. Each counter information contains the number of GTs, the number of octets, an eventual data problem and the selectionGroup for which was counted. The counters are not readable but only available in the notification data.

If all counters for verification or accounting should still be zero at the end of the measurement, the corresponding notification should be generated regardless of that in order to provide a measure against notification loss.

Two different notifications, one for accounting, one for accounting verification containing the measurement results might be generated by this one object. If a selectionGroupSet is empty, then this means, that accounting resp. verification is not performed for the adjacent operator. In this case there is no notification for accounting resp. verification.

The following rules apply for set requests on the sccpLinkageSet, selectionGroupSetForAccounting and selectionGroupSetForVerification attributes: The addressed sccpLinkages in the sccpLinkageSet attribute may only appear in one sccpAccount. The selectionGroups in the selectionGroupSetForVerification shall refer to terminatingAccountClassForVerification instances contained in this sccpAccount instance. The selectionGroups in the selectionGroupSetForAccounting shall refer to terminatingAccountClass instancesForAccounting.";;

ATTRIBUTES

sccpAccountId  GET SET-BY-CREATE,

sccpLinkageSet  GET SET-BY-CREATE ADD-REMOVE,

operatorName  GET SET-BY-CREATE,

selectionGroupSetForAccounting 	INITIAL VALUE selectionGroupSetInitial GET REPLACE,

selectionGroupSetForVerification 	INITIAL VALUE selectionGroupSetInitial GET REPLACE;

REGISTERED AS {sccpAccountPackage-POi};

sccpAccountingLogRecordPackagePACKAGE

BEHAVIOUR sccpAccountingLogRecordPackageBehaviour BEHAVIOUR DEFINED AS

"The sccpAccountingLogRecord managed object is used to represent logged information that resulted from the sccpAccounting or sccpAccountingVerification notifications.";;

ATTRIBUTES

endOfMeasurementTime 		GET,

sccpLinkageSet 			GET,

sccpAccCounterDataSequence 	GET;

REGISTERED AS {sccpAccountingLogRecordPackage-POi};

sccpAccountingNotificationsPackage PACKAGE

BEHAVIOUR sccpAccountingNotificationsPackagePackageBehaviour BEHAVIOUR DEFINED AS

"The sccpAccountingLogRecord managed object is used to represent logged information that resulted from the sccpAccounting or sccpAccountingVerification notifications.";;

NOTIFICATIONS

sccpAccounting,

sccpAccountingVerification;

REGISTERED AS { sccpAccountingNotificationsPackage-POi};

terminatingAccountClassForAccountingPackage PACKAGE

BEHAVIOUR terminatingAccountClassForAccountingBehaviour BEHAVIOUR DEFINED AS

"The terminatingAccountClassForAccounting instances contain a set of gtRules with the same tariff. The terminating account class for accounting is identified by the terminatingAccountClassForAccountingId. An instance of this class represents the destination information item involved in accounting.

The following restrictions apply for set requests on the ruleSet attribute: A rule may be contained in only one ruleSet within the scope of the superior scrc.";;

ATTRIBUTES

terminatingAccountClassForAccountingId 	GET SET-BY-CREATE,

ruleSet 						GET SET-BY-CREATE ADD-REMOVE;

REGISTERED AS {terminatingAccountClassForAccountingPackage-POi};

terminatingAccountClassForVerificationPackage PACKAGE

BEHAVIOUR terminatingAccountClassForVerificationBehaviour BEHAVIOUR DEFINED AS

"The terminatingAccountClassForVerification instances contain a set of verificationRules with the same tariff. The terminating account class for verification is identified by the terminatingAccountClassForAccountingId. An instance of this class represents the destination information item involved in accounting verification.

The following restrictions apply for set requests on the ruleSet attribute: A rule may be contained in only one ruleSet within the scope of a superior sccpAccount.";;

ATTRIBUTES

terminatingAccountClassForVerificationId 	GET SET-BY-CREATE,

ruleSet 						GET SET-BY-CREATE ADD-REMOVE;

REGISTERED AS {terminatingAccountClassForVerificationPackage-POi};

verificationRulePackage PACKAGE

BEHAVIOUR verificationRuleBehaviour BEHAVIOUR DEFINED AS

"The verificationRules are selected during the translation of outgoing global titles for accounting verification purposes. As the translation mechanism is the same as the translation for routing purposes, the same selection attributes are applicable: gtAddressInformation and possibly the encodingScheme. If there is no matching verificationRule found, no terminatingAccountClassForVerification can be selected. In this case, no accounting verification counter is to be incremented.";;

ATTRIBUTES

verificationRuleId						GET SET-BY-CREATE,

ITU-T Rec. Q.751.2 (1997):gtAddressInformation	GET-REPLACE,

ITU-T Rec. Q.751.2 (1997):gtEncodingScheme	GET SET-BY-CREATE;

REGISTERED AS {verificationRulePackage-POi};

verificationTranslatorPackage PACKAGE

BEHAVIOUR verificationTranslatorBehaviour BEHAVIOUR DEFINED AS

"The verificationTranslators are selected during the translation of outgoing global titles for accounting verification purposes. As the translation mechanism is the same as the translation for routing purposes, the same selection attributes are applicable: gtIndicator, gtNatureOfAddress, gtNumberingPlan and gtTranslationType. No single-valued address attributes are choosen however. This is to avoid that identical sets of verificationRule-sets have to be created in case of identical routing for a set of address parameter values. A verification translator, if selected, specifies a set of contained verificationRules that apply for a certain outgoing global title.";;

ATTRIBUTES

verificationTranslatorId 	GET SET-BY-CREATE,

ITU-T Rec. Q.751.2 (1997):gtIndicator	GET SET-BY-CREATE,

gtNatureOfAddressSet	GET-REPLACE ADD-REMOVE,

gtNumberingPlanSet	GET-REPLACE ADD-REMOVE,

gtTranslationTypeSet	GET-REPLACE ADD-REMOVE;

REGISTERED AS {verificationTranslatorPackage-POi};

### 7.2.3	Attribute definitions

ruleSet ATTRIBUTE

WITH ATTRIBUTE SYNTAX SccpAccountingDefinedTypesModule.RuleSet;

MATCHES FOR EQUALITY,SET-COMPARISON,SET-INTERSECTION;

BEHAVIOUR gtRuleSetBehaviour BEHAVIOUR DEFINED AS

"The ruleSet attribute refers to a set (at least one) of gtRules or verificationRules, all belonging to the same terminating account class.

A set request is rejected if

a rule would be referenced which is already referenced by another terminatingAccountClass(ForAccounting/Verification) contained in the same superior object class instance.";;

REGISTERED AS {ruleSet-AOi};

sccpAccountId ATTRIBUTE

WITH ATTRIBUTE SYNTAX SccpAccountingDefinedTypesModule.SimpleNameType;

MATCHES FOR EQUALITY;

BEHAVIOUR sccpAccountIdBehaviour BEHAVIOUR DEFINED AS

"The sccpAccountId is the naming attribute of the sccpAccount managed object.";;

REGISTERED AS {sccpAccountId-AOi};

sccpLinkageSet ATTRIBUTE

WITH ATTRIBUTE SYNTAX SccpAccountingDefinedTypesModule.SccpLinkageSet;

MATCHES FOR EQUALITY,SET-COMPARISON,SET-INTERSECTION;

BEHAVIOUR sccpLinkageSetBehaviour BEHAVIOUR DEFINED AS

"The sccpLinkageSet refers to a set (at least one) of sccpLinkages, identifying an operator.";;

REGISTERED AS {sccpLinkageSet-AOi};

terminatingAccountClassForAccountingId ATTRIBUTE

WITH ATTRIBUTE SYNTAX SccpAccountingDefinedTypesModule.SimpleNameType;

MATCHES FOR EQUALITY;

BEHAVIOUR terminatingAccountClassForAccountingIdBehaviour BEHAVIOUR DEFINED AS

"The terminatingAccountClassForAccountingId is the naming attribute of the terminatingAccountClassForAccounting managed object. ";;

REGISTERED AS {terminatingAccountClassForAccountingId-AOi};

terminatingAccountClassForVerificationId ATTRIBUTE

WITH ATTRIBUTE SYNTAX SccpAccountingDefinedTypesModule.SimpleNameType;

MATCHES FOR EQUALITY;

BEHAVIOUR terminatingAccountClassForVerificationIdBehaviour BEHAVIOUR DEFINED AS

"The terminatingAccountClassForVerificationId is the naming attribute of the terminatingAccountClassForVerification managed object. ";;

REGISTERED AS {terminatingAccountClassForVerificationId-AOi};

verificationTranslatorId ATTRIBUTE

WITH ATTRIBUTE SYNTAX SccpAccountingDefinedTypesModule.SimpleNameType;

MATCHES FOR EQUALITY;

BEHAVIOUR verificationTranslatorIdBehaviour BEHAVIOUR DEFINED AS

"The verificationTranslatorId is the naming attribute of the verificationTranslator managed object.";;

REGISTERED AS {verificationTranslatorId-AOi};

verificationRuleId ATTRIBUTE

WITH ATTRIBUTE SYNTAX SccpAccountingDefinedTypesModule.SimpleNameType;

MATCHES FOR EQUALITY;

BEHAVIOUR verificationRuleIdBehaviour BEHAVIOUR DEFINED AS

"The verificationRuleId is the naming attribute of the verificationRule managed object.";;

REGISTERED AS {verificationRuleId-AOi};

gtNatureOfAddressSet ATTRIBUTE

WITH ATTRIBUTE SYNTAX SccpAccountingDefinedTypesModule.GtNatureOfAddressSet;

MATCHES FOR EQUALITY,SET-COMPARISON,SET-INTERSECTION;

BEHAVIOUR gtNatureOfAddressSetBehaviour BEHAVIOUR DEFINED AS

"The gtNatureOfAddressSet attribute defines a set of gtNatureOfAddress values.";;

REGISTERED AS {gtNatureOfAddressSet-AOi}

gtNumberingPlanSet ATTRIBUTE

WITH ATTRIBUTE SYNTAX SccpAccountingDefinedTypesModule.GtNumberingPlanSet;

MATCHES FOR EQUALITY,SET-COMPARISON,SET-INTERSECTION;

BEHAVIOUR gtNumberingPlanSetBehaviour BEHAVIOUR DEFINED AS

"The gtNumberingPlanSet attribute defines a set of gtNumberingPlan values.";;

REGISTERED AS {gtNumberingPlanSet-AOi}

gtTranslationTypeSet ATTRIBUTE

WITH ATTRIBUTE SYNTAX SccpAccountingDefinedTypesModule.GtTranslationTypeSet;

MATCHES FOR EQUALITY,SET-COMPARISON,SET-INTERSECTION;

BEHAVIOUR gtTranslationTypeSetBehaviour BEHAVIOUR DEFINED AS

"The gtTranslationTypeSet attribute defines a set of gtTranslationType values.";;

REGISTERED AS {gtTranslationType-AOi}

### 7.2.4	Name binding definitions

sccpAccount-scrc NAME BINDING

SUBORDINATE OBJECT CLASS sccpAccount AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS "ITU-T Rec. Q.751.2 (1996)":scrc AND SUBCLASSES;

WITH ATTRIBUTE sccpAccountId;

BEHAVIOUR sccpAccount-scrcBehaviour BEHAVIOUR DEFINED AS

"This name binding is used when the sccpAccount instance is created by management operations.

A create request is rejected if

at least one of the instances which would be referenced by the attribute sccpLinkageSet is not existing

OR

at least one of the instances which would be referenced by the attributes selectionGroupSetForAccounting or selectionGroupSetForVerification is not existing.";;

CREATE;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {sccpAccount-scrc-NBOI};

terminatingAccountClassForAccounting-scrc NAME BINDING

SUBORDINATE OBJECT CLASS terminatingAccountClassForAccounting AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS "ITU-T Rec. Q.751.2 (1996)":scrc AND SUBCLASSES;

WITH ATTRIBUTE terminatingAccountClassForAccountingId;

BEHAVIOUR terminatingAccountClassForAccounting-scrcBehaviour BEHAVIOUR DEFINED AS

"This name binding is used when a terminatingAccountClassForAccounting instance for SCCP accounting is created by management operations.

A create request is rejected if

a gtRule would be referenced via the attribute ruleSet which is already referenced by another terminatingAccountClassForAccounting contained in the same superior object class instance.

A delete request is rejected if

the instance is still referenced by an instance of object class sccpAccount via attribute selectionGroupSetForAccounting.";;

CREATE WITH REFERENCE OBJECT INSTANCE;

DELETE;

REGISTERED AS {terminatingAccountClassForAccounting-scrc-NBOI};

terminatingAccountClassForVerification-sccpAccount NAME BINDING

SUBORDINATE OBJECT CLASS terminatingAccountClassForVerification AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS sccpAccount AND SUBCLASSES;

WITH ATTRIBUTE terminatingAccountClassForVerificationId;

BEHAVIOUR terminatingAccountClassForVerification-sccpAccountBehaviour BEHAVIOUR DEFINED AS

"This name binding is used when a terminatingAccountClassForVerification instance for SCCP accounting verification is created by management operations.

A create request is rejected if

a verificationRule would be referenced via the attribute ruleSet which is already referenced by another terminatingAccountClassForVerification contained in the same superior object class instance,

or if

a verificationRule would be referenced via the attribute ruleSet which is contained in another sccpAccount instance than the terminatingAccountClassForVerification

A delete request is rejected if

the instance is still referenced by an instance of object class sccpAccount via attribute selectionGroupSetForVerification.";;

CREATE WITH REFERENCE OBJECT INSTANCE;

DELETE;

REGISTERED AS {terminatingAccountClassForVerification-sccpAccount-NBOI};

verificationTranslator-sccpAccount NAME BINDING

SUBORDINATE OBJECT CLASS verificationTranslator AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS sccpAccount AND SUBCLASSES;

WITH ATTRIBUTE verificationTranslatorId;

BEHAVIOUR verificationTranslator-sccpAccount-Bhv BEHAVIOUR DEFINED AS

"This name binding is used when a verificationTranslator instance for SCCP accounting is created by management operations.";;

CREATE WITH REFERENCE OBJECT INSTANCE;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {verificationTranslator-sccpAccount-NBOI};

verificationRule-verificationTranslator NAME BINDING

SUBORDINATE OBJECT CLASS verificationRule AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS verificationTranslator AND SUBCLASSES;

WITH ATTRIBUTE verificationRuleId;

BEHAVIOUR verificationRule-verificationTranslator-Bhv BEHAVIOUR DEFINED AS

"This name binding is used when a verificationRule instance for SCCP accounting is created by management operations.";;

CREATE WITH REFERENCE OBJECT INSTANCE;

DELETE;

REGISTERED AS {verificationRule-verificationTranslator-NBOI};

### 7.2.5	Notification definitions

sccpAccounting NOTIFICATION

BEHAVIOUR sccpAccountingBehaviour BEHAVIOUR DEFINED AS

"This notification is generated on occurrence of the event specified in the reporting triggers attribute of the ss7AccountingAndVerificationControl object controlling the sccpAccount, except if the attribute selectionGroupSetForAccounting of the instance has size zero. The notification shall also be sent, if all counters have the value zero.
If accounting is done for all accounts in the same way, then it is possible to include the ruleSet only in one notification (e.g. the first) of the interval and omit it in all others. In this case the sequence of the given counters must be identical, i.e. refer to the same gtRules, for all notifications of the interval.";;

WITH INFORMATION SYNTAX SccpAccountingDefinedTypesModule.SccpAccountingNotificationData

AND ATTRIBUTE IDS

endOfMeasurementTime		endOfMeasurementTime,

sccpLinkageSet 			sccpLinkageSet,

sccpAccCounterDataSequence	sccpAccCounterDataSequence;

REGISTERED AS {sccpAccounting-NOI}

sccpAccountingVerification NOTIFICATION

BEHAVIOUR sccpAccountingVerificationBehaviour BEHAVIOUR DEFINED AS

"This notification is generated on occurrence of the event specified in the reporting triggers attribute of the ss7AccountingAndVerificationControl object controlling the sccpAccount, except if the attribute selectionGroupSetForVerification of the instance has size zero. The notification shall also be sent, if all counters have the value zero.
If verification is done for all accounts in the same way, then it is possible to include the ruleSet only in one notification (e.g. the first) of the interval and omit it in all others. In this case the sequence of the given counters must be identical, i.e. refer to the same gtRules, for all notifications of the interval.";;

WITH INFORMATION SYNTAX
SccpAccountingDefinedTypesModule.SccpAccountingNotificationData

AND ATTRIBUTE IDS

endOfMeasurementTime		endOfMeasurementTime,

sccpLinkageSet 			sccpLinkageSet,

sccpAccCounterDataSequence	sccpAccCounterDataSequence;

REGISTERED AS {sccpAccountingVerification-NOI}

### 7.2.6	Parameter definitions

Currently none defined.

### 7.2.7	ASN.1 definitions

SccpAccountingDefinedTypesModule

{itu-t(0) recommendation(0) q(17) omap(751) sccpAccounting(4) informationModel(0) asn1Modules(2) sccpAccountingDefinedTypesModule(0)}

DEFINITIONS IMPLICIT TAGS ::= BEGIN

IMPORTS

ObjectInstance, SimpleNameType

FROM Attribute-ASN1Module {joint-iso-itu-t ms(9) smi(3) part2(2) asn1Module(2) 1}

Counter, DataProblem, accountingInformationModel, accountingAction, accountingAttribute, accountingAttributeGroup, accountingNameBinding, accountingNotification, accountingObjectClass, accountingPackage, accountingParameter, SelectionGroupSequence, SelectionGroup
FROM AccountingDefinedTypesModule {itu-t(0) recommendation(0) q(17) omap(751) accounting(3) informationModel(0) asn1Modules(2) accountingDefinedTypesModule(0)}

GtNatureOfAddress,GtNumberingPlan,GtTranslationType

FROM SCCPDefinedTypesModule{itu-t(0) recommendation q(17) omap(751) sccp(2) informationModel(0) asn1Modules(2) sccpDefinedTypesModule(0)}

Ss7SpecificErrorInformation

FROM {itu(0) recommendation(0) q(17) omap2(2751) part1(1) informationModel(0) asn1Modules(2) q2751DefinedTypesModule(0)}

EXPORTS EVERYTHING

-- ASN.1 TYPE DEFINITIONS

RuleSet ::= SET SIZE (1..maxNumberReferencesInRuleSet) OF ObjectInstance

maxNumberReferencesInRuleSet INTEGER::= i -- this number is only for compilability

maxNumberReferencesInSccpLinkageSet INTEGER::= j -- this number is only for compilability

maxNumberReferencesInSccpSelectionGroupSet INTEGER::= q -- this number is only for compilability

SccpLinkageSet ::= SET SIZE (1..maxNumberReferencesInSccpLinkageSet ) OF ObjectInstance

SccpAccCounterData ::= SEQUENCE

{gts		[0] INTEGER,

octetts		[1] INTEGER,

dataProblem	[2] DataProblem,

ruleSet	[3] RuleSet OPTIONAL}

SccpAccCounterDataSequence ::= SEQUENCE SIZE
(1..maxNumberReferencesInSccpSelectionGroupSet} OF SccpAccCounterData -- maximum size = q

SccpAccountingNotificationData ::= SEQUENCE {

endOfMeasurementTime 	EndOfMeasurementTime,

sccpLinkageSet 	SccpLinkageSet,

sccpAccCounterData Sequence 	SccpAccCounterDataSequence}

selectionGroupSetInitial SelectionGroupSet ::= {}

GtNatureOfAddressSet ::= SET OF GtNatureOfAddress

GtNumberingPlanSet ::= SET OF GtNumberingPlan

GtTranslationTypeSet ::= SET OF GtTranslationType

-- the following values of Ss7SpecificErrorInformation defined in Q.2751.1 are used:

gtRuleAlreadyUsedByAnotherTAC Ss7SpecificErrorInformation ::= 4000

-- One of the gtRules specified in attribute ruleSet is already

-- used by another terminatingAccountClassForAccounting.

invalidTACForAccountingReference Ss7SpecificErrorInformation ::= 4001

-- at least one of the references in selectionGroupSetForAccounting is not

-- referring to a terminatingAccountClassForAccounting

invalidTACForVerificationReference Ss7SpecificErrorInformation ::= 4002

-- at least one of the references in selectionGroupSetForVerification is not

-- referring to a terminatingAccountClassForVerification contained in the -- same 					-- sccpAccount.

ruleOverlapError Ss7SpecificErrorInformation ::= 4003

-- the ruleSet of the-terminatingAccountClassForAccounting/Verification

-- is not disjunct ! No unambiguous identification of a

-- terminatingAccountClass would be possible.

sccpLinkageAlreadyInOtherAccount Ss7SpecificErrorInformation ::= 4004

-- at least one of the sccpLinkages refered to by the sccpLinkageSet is

-- already -referred to by another sccpLinkageSet in another sccpAccount

-- instance

selectionGroupOverlapError Ss7SpecificErrorInformation ::= 4005

-- the manipulated selectionGroupSetForAccounting/Verification would

-- not allow an unambigous identification of a the counter to be

-- incremented

-- ASN.1 OBJECT IDENTIFIER definitions

ruleSet-AOi OBJECT IDENTIFIER ::= {accountingAttribute gtRuleSet(22)}

sccpAccCounterDataSequence-AOi OBJECT IDENTIFIER ::= {accountingAttribute sccpAccCounterDataSequence(26)

sccpAccountId-AOi OBJECT IDENTIFIER ::= {accountingAttribute sccpAccountId(23)}

sccpAccounting-NOI OBJECT IDENTIFIER::={accountingNotification sccpAccounting (3)}

sccpAccountingVerification-NOI OBJECT IDENTIFIER ::= {accountingNotification sccpAccountingVerification(4) }

sccpAccountPackage-POi OBJECT IDENTIFIER ::= {accountingPackage sccpAccountPackage(12)}

sccpAccount-OOi OBJECT IDENTIFIER ::= {accountingObjectClass sccpAccount(11)}

sccpAccountingLogRecord-OOi OBJECT IDENTIFIER ::= {accountingObjectClass sccpAccountingLocRecord(7)}

sccpAccountingLogRecordPackage-POi OBJECT IDENTIFIER ::= {accountingPackage

sccpAccountingLogRecordPackage (13)}

sccpAccountingNotificationsPackage-POi OBJECT IDENTIFIER ::= {accountingPackage sccpAccountingNotificationsPackage(11)}

sccpLinkageSet-AOi OBJECT IDENTIFIER ::= {accountingAttribute sccpLinkageSet(24)}

sccpAccount-scrc-NBOI OBJECT IDENTIFIER ::= {accountingNameBinding sccpAccount-scrc(6)}

terminatingAccountClassForVerification-sccpAccount-NBOI OBJECT IDENTIFIER ::= {accountingNameBinding terminatingAccountClassForVerification-sccpAccount(7)}

terminatingAccountClassForAccounting-scrc-NBOI OBJECT IDENTIFIER ::=
{accountingNameBinding terminatingAccountClassForAccounting-scrc(8)}

terminatingAccountClassForAccountingId-AOi OBJECT IDENTIFIER ::=
{accountingAttribute terminatingAccountClassForAccountingId(25)}

terminatingAccountClassForVerificationId-AOi OBJECT IDENTIFIER ::=
{accountingAttribute terminatingAccountClassForVerificationId(21)}

terminatingAccountClassForAccountingPackage-POi OBJECT IDENTIFIER ::=
{accountingPackage terminatingAccountClassForAccountingPackage (7)}

terminatingAccountClassForVerificationPackage-POi OBJECT IDENTIFIER ::=
{accountingPackage terminatingAccountClassForVerificationPackage (10)}

terminatingAccountClassForAccounting-OOi OBJECT IDENTIFIER ::=
{accountingObjectClass terminatingAccountClassForAccounting (6)}

terminatingAccountClassForVerification-OOi OBJECT IDENTIFIER ::=
{accountingObjectClass terminatingAccountClassForVerification (10)}

verificationRule-OOi OBJECT IDENTIFIER ::= {accountingObjectClass
verificationRule(9)}

verificationTranslator-OOi OBJECT IDENTIFIER ::= {accountingObjectClass
verificationTranslator(8)}

verificationRulePackage-POi OBJECT IDENTIFIER ::= {accountingPackage 
verificationRulePackage(8)}

verificationTranslatorPackage-POi OBJECT IDENTIFIER ::= {accountingPackage
verificationTranslatorPackage(9)}

verificationTranslatorId-AOi OBJECT IDENTIFIER ::= {accountingAttribute
verificationTranslatorId(16)}

verificationRuleId-AOi OBJECT IDENTIFIER ::= {accountingAttribute
verificationRuleId(17)}

gtNatureOfAddressSet-AOi OBJECT IDENTIFIER ::= {accountingAttribute
gtNatureOfAddressSet(18)}

gtNumberingPlanSet-AOi OBJECT IDENTIFIER ::= {accountingAttribute
gtNumberingPlanSet(19)}

gtTranslationTypeSet-AOi OBJECT IDENTIFIER ::= {accountingAttribute
gtTranslationTypeSet(20)}

verificationTranslator-sccpAccount-NBOi OBJECT IDENTIFIER ::=
{accountingNameBindingverificationTranslator-sccpAccount(9)}

verificationRule-verificationTranslator-NBOi OBJECT IDENTIFIER ::=
{accountingNameBinding verificationRule-verificationTranslator(10)}

END