SCF Postcondition:

–	SLPI execution may continue.

In the SCSM-FSM state "Preparing SSF Instructions", this operation is invoked by the SCF if the service logic results in the request to the SSF to route a call to a specific destination. If no event monitoring has been requested and no reports (CallInformationReport and ApplyChargingReport) have been requested in a previously sent operation, a SCSM-FSM transition to state "Idle" occurs. Otherwise, if event monitoring has been requested or any report (CallInformationReport and ApplyChargingReport) has been requested, the SCSM-FSM transitions to state "Waiting for Notification or Report". When the 'Connect' operation is used in the context of a hand-off procedure, the SCSM-FSM transitions to state "Idle". However, in this case, the SCF must maintain sufficient information in order to correlate the subsequent 'AssistRequestInstructions' operation (from the assisting SSF or SRF) to the existing SLPI.

##### 3.3.16.2.2	Error handling

If reject or error messages are received, then the SCSM informs the SLPI and remains in the state "Preparing SSF Instructions".

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

#### 3.3.16.3	Responding Entity (SSF)

##### 3.3.16.3.1	Normal procedure

SSF Preconditions:

1)	Call origination attempt has been initiated.

2)	Basic call processing has been suspended at a DP.

3)	The SSF waits for instructions.

SSF Postconditions:

1)	The SSF performs the call processing actions to route the call to the specified destination.

2)	In the O-BCSM, when only address information is included in the Connect operation, call processing resumes at PIC 3.

3)	In the O-BCSM, when address information and routing information is included in the Connect operation, call processing resumes at PIC 4.

On receipt of this operation in the SSF-FSM state "Waiting for Instructions", the SSP performs the following actions:

–	The SSF cancels TSSF.

–	If "cutAndPaste" is present, then the SSF deletes ("cut") from the dialled IN number the indicated number of digits and pastes the remaining dialled digits at the end of the "destinationRoutingAddress" parameter delivered by the SCF. The resulting directory number is used for routing to complete the related call.

–	If "cutAndPaste" is not present, then the "destinationRoutingAddress" parameter delivered by the SCF is used for routing to complete the related call. Note that in the case of hand-off, this results in routing to an assisting SSP or IP.

–	If the "callingPartyNumber" is supplied, this value may be used for all subsequent SSF processing.

–	If no EDPs have been armed and neither a CallInformationReport nor an ApplyChargingReport has been requested, the FSM goes to state "Idle" (e9). Otherwise, the FSM goes to state "Monitoring" (e11).

No implicit activation or deactivation of DPs occurs.

Statistic counter(s) are not affected.

Connect completes when the INAP processing of the operation is complete and before the SSP starts the processing necessary to select a circuit.

Therefore, in order to detect route select failure after a "Connect" it is necessary to explicitly arm the "Route Select Failure" EDP before sending the "Connect" (although they may be in the same message).

##### 3.3.16.3.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

### 3.3.17	ConnectToResource procedureXE "ConnectToResource procedure"§

Recommendation Q.1218     (10/95)

#### 3.3.17.1	General description

This operation is used to connect a call from the SSF to a specialized resource. After successful connection to the SRF, the interaction with the caller can take place. The SSF relays all operations for the SRF and all responses from the SRF.

##### 3.3.17.1.1	Parameters

·	resourceAddress:

This parameter identifies the physical location of the SRF.

–	iPRoutingAddress:

This parameter indicates the routing address to set up a connection towards the SRF.

–	none:

This parameter indicates that the call party is to be connected to a predefined SRF.

·	serviceInteractionIndicators:

This parameter contain indicators sent from the SCP to the SSP for control of the network based services at the originating exchange and the destination exchange.

#### 3.3.17.2	Invoking Entity (SCF)

##### 3.3.17.2.1	Normal procedure

SCF Preconditions:

1)	A control relationship exists between the SCF and the SSF.

2)	The SLPI has determined that additional information from the call party is needed.

3)	The call party is not connected to any other party.

4)	The SCSM-FSM is in the state "Routing to Resource", substate "Determine Mode".

5)	The SLPI has determined that the SRF can be accessed from the SSF.

SCF Postconditions:

1)	The SCSM sends out a 'PlayAnnouncement' or 'PromptAndCollectUserInformation' operation accompanying the 'ConnectToResource'

2)	The SCSM-FSM moves to the state "User Interaction".

##### 3.3.17.2.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

#### 3.3.17.3	Responding Entity (SSF)

##### 3.3.17.3.1	Normal procedure

SSF Preconditions:

1)	Basic call processing has been suspended at a DP and a control relationship has been established.

2)	The SSF-FSM is in the state "Waiting for Instructions".

SSF Postconditions:

1)	The call is switched to the SRF.

2)	A control relationship to the SRF is established.

3)	The SSF-FSM moves to the state "Waiting for End of User Interaction". If necessary, TSSF is set.

NOTES

1	Whether the TSSF is used or not in this case is network operator dependent. But it must be synchronized with TSCF-SSF in the SCSM.

2	The successful connection to the SRF causes a state transition in the SRF-FSM from "Idle" to "Connected".

##### 3.3.17.3.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

### 3.3.18	Continue procedureXE "Continue procedure"§

#### 3.3.18.1	General description

This operation is used to request the SSF to proceed with call processing at the DP at which it previously suspended call processing to await SCF instructions. The SSF continues call processing without substituting new data from the SCF.

##### 3.3.18.1.1	Parameters

None.

#### 3.3.18.2	Invoking Entity (SCF)

##### 3.3.18.2.1	Normal procedure

SCF Precondition:

–	SCSM is in the state "Preparing SSF instructions".

SCF Postcondition:

–	SCSM is in the state "Waiting for Notification or Request", in case monitoring was required, or in the state "Idle", in case no monitoring was required.

The SCSM is in state "Preparing SSF instructions". The 'Continue' operation is invoked by a SLPI. This causes a SCSM transition to state "Idle" if no subsequent monitoring is required. However, if monitoring is required, like in the case of armed EDPs or outstanding report requests, the SCSM transitions to state "Waiting for Notification of Report".

##### 3.3.18.2.2	Error handling

Operation related error handling is not applicable, due to class 4 operation.

#### 3.3.18.3	Responding Entity (SSF)

##### 3.3.18.3.1	Normal procedure

SSF Preconditions:

1)	BCSM: Basic call processing has been suspended at any DP.

2)	SSF-FSM is in the state "Waiting for Instructions".

SSF Postconditions:

1)	BCSM: Basic call processing continues.

2)	SSF-FSM is in the state "Monitoring", because at least one EDP was armed, or a 'CallInformationReport' or 'ApplyChargingReport' was requested; or

SSF-FSM is in the state "Idle", because no EDPs were armed and neither the 'CallInformationReport' nor the 'ApplyChargingReport' was requested.

The SSF-FSM is in state "Waiting for instructions". The SSME-Control receives the 'Continue' operation and relays it to the appropriate SSF-FSM. The SSF-FSM transitions to state "Idle" in case no EDPs are armed and no outstanding report requests are present. The SSF-FSM transits to state "Monitoring" if at least one EDP is armed, or if there is at least one outstanding report request. Basic call processing is resumed.

##### 3.3.18.3.2	Error handling

Operation related error handling is not applicable, due to class 4 operation.

### 3.3.19	DisconnectForwardConnection procedureXE "DisconnectForwardConnection procedure"§

#### 3.3.19.1	General description

This operation is used in the following two cases:

1)	To clear a connection to a SRF

This operation is used to explicitly disconnect a connection to a resource (SRF) established previously with a 'ConnectToResource' or an 'EstablishTemporaryConnection' operation. It is used for a forward disconnection from the SSF. An alternative solution is the backward disconnect from the SRF, controlled by the 'DisconnectFromIPForbidden' parameter in the 'PlayAnnouncement' and 'PromptAndCollectUserInformation' operations.

2)	To clear a connection to an assisting SSF

This operation is sent to the non-assisting SSF of a pair of SSFs involved in an assist procedure. It is used to disconnect the temporary connection between the initiating SSF and the assisting SSF, and the assisting SSF and its associated SRF.

##### 3.3.19.1.1	Parameters

None.

#### 3.3.19.2	Invoking Entity (SCF)

##### 3.3.19.2.1	Normal procedure

SCF Preconditions:

1)	A control relationship exists between the SCF and the SSF.

2)	An assist- or a relay procedure is in progress.

3)	An SLPI has determined that a 'DisconnectForwardConnection' operation has to be sent by the SCF.

SCF Postcondition:

–	SLPI execution may continue.

The 'DisconnectForwardConnection' operation is used to instruct the SSF to disconnect the concerned forward connection to the assisting SSF or the physical entity containing the SRF.

In the SCSM-FSM state "User Interaction", substate "Waiting for Response from the SRF", this operation is invoked by the SCF when the service logic determines that user interaction is finished and requests the SSF to disconnect the temporary connection to the assisting SSF or the SRF. The SCSM-FSM then transitions to state "Preparing SSF Instructions".

##### 3.3.19.2.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

#### 3.3.19.3	Responding Entity (SSF)

##### 3.3.19.3.1	Normal procedure

SSF Preconditions:

1)	Call origination attempt has been initiated.

2)	Basic call processing has been suspended at a DP.

3)	The initiating SSF is in the state "Waiting for End of User Interaction" or "Waiting for End of Temporary Connection".

SSF Postconditions:

1)	The connection to the SRF or assisting SSF is released.

2)	The SSF is waiting for instructions.

The receipt of "DisconnectForwardConnection" results in disconnecting the assisting SSF or the physical entity containing the SRF from the concerned call. It does not release the connection from the SSF back to the end user.

This operation is accepted in the SSF-FSM states "Waiting for End of Temporary Connection" or "Waiting for End of User Interaction". On receipt of this operation in these states, the SSF must perform the following actions:

–	The initiating SSF releases the connection to the assisting SSF or the relay SRF.

–	The SSF resets TSSF.

–	The SSF-FSM goes to state "Waiting for Instructions" (e8).

NOTE – The successful disconnection to the SRF causes a state transition in the SRF-FSM to "Idle". A current order ("PlayAnnouncement" or "PromptAndCollectUserInformation") is cancelled and any queued order ("PlayAnnouncement" or "PromptAndCollectUserInformation") is discarded.

##### 3.3.19.3.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

### 3.3.20	EstablishTemporaryConnection procedureXE "EstablishTemporaryConnection procedure"§

#### 3.3.20.1	General description

This operation is used to create a connection between an initiating SSF and an assisting SSF as part of a service assist procedure. It can also be used to create a connection between a SSF and a SRF, for the case where the SRF exists in a separately addressable physical entity.

##### 3.3.20.1.1	Parameters

–	assistingSSPIPRoutingAddress:

This parameter indicates the destination address of the SRF for assist procedure. The 'assistingSSPIPRoutingAddress' may contain embedded within it, a 'correlationID' and 'scfID', but only if 'correlationID' and 'scfID' are not specified separately.

–	correlationID:

This parameter is used by the SCF to associate the 'AssistRequestInstructions' from the assisting SSF (or the SRF) with the Request from the initiating SSF. The 'correlationID' is used only if the correlation id is not embedded in the 'assistingSSPIPRoutingAddress'. The network operator has to decide about the actual mapping of this parameter on the used signalling system.

–	scfID:

See Recommendation Q.1290. The 'scfID' is used only if the SCF id is not embedded in the 'assistingSSPIPRoutingAddress'. The network operator has to decide about the actual mapping of this parameter on the used signalling system.

–	carrier:

See Recommendation Q.1290. In this message, the carrier selection field is null (00000000) and Carrier ID indicates the carrier to use for the call.

–	serviceInteractionIndicators:

This parameter contain indicators sent from the SCP to the SSP for control of the network based services at the originating exchange and the destination exchange.

#### 3.3.20.2	Invoking Entity (SCF)

##### 3.3.20.2.1	Normal procedure

SCF Preconditions:

1)	A control relationship exists between the SCF and the SSF.

2)	The service logic has determined that a connection is needed between the SSF and SRF or between the SSF and an assisting SSF.

3)	The call party is not connected to any other party.

SCF Postcondition:

–	The SCF is "Waiting for Assist Request Instructions".

In the SCSM-FSM state "Routing to Resource", this operation is invoked by the SCF when the service logic determines that an assisting SSF or a Direct SCF-SRF relation is needed. The SCSM-FSM then transitions to state "Waiting for Assist Request Instructions".

##### 3.3.20.2.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

#### 3.3.20.3	Responding Entity (SSF)

##### 3.3.20.3.1	Normal procedure

SSF Preconditions:

1)	Call origination attempt has been initiated.

2)	Basic call processing has been suspended at DP 1,2,3,4,5,6,7,12,13,14, or 15.

3)	The SSF waits for instructions.

4)	The SSF is not an assisting SSF.

SSF Postconditions:

1)	The SSF performs the call processing actions to route the call to the assisting SSF or SRF according to the 'assistingSSPIPRoutingAddress' requested by the SCF.

2)	The SSF waits for end of temporary connection.

On receipt of this operation in the SSF-FSM state "Waiting for Instructions", the SSP has to perform the following actions:

–	Reset the TSSF (optional).

NOTE – This "optional" means that the application timer TSSF is optionally set. Whether it is used or not is network operator dependent. But it must be synchronized with TSCF-SSF in the SCSM.

–	Route the call to assisting SSF or SRF using 'assistingSSPIPRoutingAddress'.

–	The SSF-FSM goes to state "Waiting for End of Temporary Connection" (e7).

##### 3.3.20.3.2	Error handling

Until the connection set-up has been accepted (refer to Recommendation Q.71) by the assisting SSF-SRF, all received failure indications from the network on the ETC establishment shall be reported to the SCF as ETC error ETCFailed (e.g. busy, congestion). Note that the operation timer for ETC shall be longer than the maximum allowed time for the signalling procedures to accept the connection.

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

### 3.3.21	EventNotificationCharging procedureXE "EventNotificationCharging procedure"§

#### 3.3.21.1	General description

This operation is used by the SSF to report to the SCF the occurrence of a specific charging event type as requested by the SCF using the "RequestNotificationChargingEvent" operation. The operation supports the options to cope with the interactions concerning the charging (refer to Appendix II.4/Q.1214 "Charging scenarios").

As several charging events may occur during a connection configuration a possibility exists for the EventNotificationCharging operation to be invoked on multiple occasions. For each connection configuration EventNotificationCharging may be used several times.

##### 3.3.21.1.1	Parameters

–	eventTypeCharging:

This parameter indicates the charging event type which has occurred. Its content is network operator specific, which may be "charge pulses" or "charge messages".

–	eventSpecificInformationCharging:

This parameter contains charging related information specific to the event. Its content is network operator specific.

–	legID:

This parameter indicates the leg id on which the charging event type applies.

–	monitorMode:

This parameter indicates how the charging event is reported. When the "monitorMode" is "interrupted", the event is reported as a request; if the "monitorMode" is "notifyAndContinue" the event is reported as a notification. The "monitorMode" "transparent" is not applicable for the "EventNotificationCharging" operation.

#### 3.3.21.2	Invoking Entity (SSF)

##### 3.3.21.2.1	Normal procedure

SSF Preconditions:

1)	A control relationship exists between the SCF and the SSF.

2)	A charging event has been detected that is requested by the SCF.

SSF Postcondition:

–	No FSM state transition.

The SSF-FSM is in any state except "Idle". This operation is invoked if a charging event has been detected that is requested by the SCF. The detected charging event can be caused by:

a)	another SLPI; or

b)	another exchange.

Irrespective of the charging event cause, the SSF performs one of the following actions on occurrence of the charging event (according the corresponding monitorMode):

Interrupted:

–	Notify the SCF of the charging event using "EventNotificationCharging" operation: do not process the event, but discard it. However, call and existing charging processing will not be suspended in the SSF.

NotifyAndContinue:

–	Notify the SCF of the charging event using "EventNotificationCharging", and continue processing the event or signal.

##### 3.3.21.2.2	Error handling

Operation related error handling is not applicable, due to class 4 operation.

#### 3.3.21.3	Responding Entity (SCF)

##### 3.3.21.3.1	Normal procedure

SCF Precondition:

–	A "RequestNotificationChargingEvent" has been sent at the request of a SLPI and the SLPI is expecting an "EventNotificationCharging" from the SSF.

SCF Postcondition:

–	No FSM state transition.

On receipt of this operation the SLPI which is expecting this notification can continue. If the corresponding monitor mode was set by the SLPI to Interrupted, the SLPI prepares instructions for the SSF if necessary.

##### 3.3.21.3.2	Error handling

Operation related error handling is not applicable, due to class 4 operation.

### 3.3.22	EventReportBCSM procedureXE "EventReportBCSM procedure"§

#### 3.3.22.1	General description

This operation is used to notify the SCF of a call related event previously requested by the SCF in an 'RequestReportBCSMEvent' operation. The monitoring of more than one event could be requested with a 'RequestReportBCSMEvent' operation, but each of these requested events is reported in a separate 'EventReportBCSM' operation.

##### 3.3.22.1.1	Parameters

·	eventTypeBCSM:

This parameter specifies the type of event that is reported.

·	bcsmEventCorrelationID:

Used by the SCF for correlation with a previous operation.

·	eventSpecificInformationBCSM:

This parameter indicates the call related information specific to the event.

For 'CollectedInfo' it will contain the 'calledPartyNumber'.

For 'AnalysedInfo' it will contain the 'calledPartyNumber'.

For 'RouteSelectFailure' it will contain the 'FailureCause', if available.

For O- or T-Busy it will contain the 'BusyCause', if available.

For O- or T-NoAnswer it will be empty.

For O- or T-Answer it will be empty.

For O- or T-MidCall it will contain 'connectTime', if available.

For O- or T-Disconnect it will contain the 'releaseCause' and/or 'connectTime', if available.

The connect time, if available, indicates the duration between the received answer indication from the called party side and the release of the connection in units of 100 ms.

·	legID:

This parameter indicates the party in the call for which the event is reported. SSF will use the option 'ReceivingSideID' only.

–	receivingSideID:

The following values for 'legID' are assumed:

i)	'legID' = 1 indicates the party that was present at the moment of the 'InitialDP' or a DP-specific operation (in case of a midcall trigger, the party causing the trigger), or the party that was created with an 'InitiateCallAttempt' operation.

ii)	'legID' = 2 indicates the party that was created with a 'Connect' operation, or in case of a mid call trigger, the party not causing the trigger.

If not included, the following defaults are assumed:

iii)	legID = 1 for the events CollectedInfo, AnalysedInformation, O-Abandon and T-Abandon.

iv)	legID = 2 for the events RouteSelectFailure, O-CalledPartyBusy, O-NoAnswer, O-Answer, T-Busy, T-NoAnswer and T-Answer.

The 'legID' parameter shall always be included for the events O-MidCall, O-Disconnect, T-MidCall and T-Disconnect.

·	miscCallInfo:

This parameter indicates detection point related information.

–	messageType:

This parameter indicates whether the message is a request, i.e. resulting from a 'RequestReportBCSMEvent' with 'monitorMode' = interrupted, or a notification, i.e. resulting from a 'RequestReportBCSMEvent' with 'monitorMode' = 'notifyAndContinue'.

–	dPAssignment:

This parameter shall be omitted for this operation.

#### 3.3.22.2	Invoking Entity (SSF)

##### 3.3.22.2.1	Normal procedure

SSF Preconditions:

1)	The SSF-FSM is in the state "Monitoring"; or

the SSF-FSM may be in state "Waiting for Instructions" if the O/TDisconnect DP is armed and encountered; or

the SSF-FSM may be in any state if the O/TAbandon DP is armed and encountered.

2)	The BCSM proceeds to an EDP that is armed.

SSF Postconditions:

1)	The SSF-FSM stays in the state "Monitoring" if the message type was notification and there are still EDPs armed or a "CallInformationReport" or "ApplyChargingReport" requested.

2)	The SSF-FSM moves to the state "Idle" if the message type was notification and there are no more EDPs armed, no "CallInformationReport" or "ApplyChargingReport" are requested.

3)	The SSF-FSM moves to the state "Waiting for Instructions" if the message type was request. Call processing is interrupted.

For certain service features it is necessary that the same O-BCSM instance is reused. Examples are follow-on calls.

The decision to reuse the same O-BCSM instance can only be taken by the SCF after certain armed EDP-R’s are reported.

The considered EDP-R’s are:

–	RouteSelectFailure;

–	O_CalledPartyBusy;

–	O_NoAnswer;

–	O_Disconnect.

If a EDP-R is met that causes the release of the related leg, all EDPs related to that leg are disarmed and the event is reported via "EventReportBCSM". To allow the reuse of the same O-BCSM instance, the BCSM has to store all call related signalling parameters (e.g. "callingPartyNumber", "callingPartyCategory") until the BCSM instance is released.

In the case the respective EDP-R is met (see list above with "O-Disconnect" only for legID = 2), the A-leg will be held, the B-leg will be released and the SCF is informed via "EventReportBCSM".

If the A-party disconnects, the call is released whether or not a DP was armed.

##### 3.3.22.2.2	Error handling

In case the message type is request, on expiration of TSSF before receiving any operation, the SSF aborts the interaction with the SCF and the call is given final treatment, e.g. a final announcement.

Operation related error handling is not applicable, due to class 4 operation.

#### 3.3.22.3	Responding Entity (SCF)

##### 3.3.22.3.1	Normal procedure

SCF Preconditions:

1)	A control relationship exists between the SSF and the SCF.

2)	The SCSM-FSM is in the state "Preparing SSF Instructions", substate "Waiting for Notification or Request".

SCF Postconditions:

1)	The SCSM-FSM stays in the substate "Waiting for Notification or Request" if the message type was notification and there are still EDPs armed or a "CallInformationReport" or "ApplyChargingReport" requested; or

the SCSM-FSM moves to the state "Idle" if the message type was notification and there are no more EDPs armed, no "CallInformationReport" or "ApplyChargingReport" are requested; or

the SCSM-FSM moves to the state "Preparing SSF Instructions" if the message type was request.

2)	The event is reported to a SLPI, based on the dialogue ID. The SCF will prepare SSF or SRF instructions in accordance with the SLPI.

##### 3.3.22.3.2	Error handling

Operation related error handling is not applicable, due to class 4 operation.

### 3.3.23	FurnishChargingInformation procedureXE "FurnishChargingInformation procedure"§

#### 3.3.23.1	General description

This operation is used to request the SSF to generate, register a call record or to include some information in the default call record. The registered call record is intended for off-line charging of the call. A possibility exists for the FurnishChargingInformation (FCI) operation to be invoked on multiple occasions. FCI could be applied at the beginning of the call in order to request to start call record generation. In addition FCI can also be applied at the end of the call or connection configuration (e.g. for follow-on calls). In this case FCI is used to include charge related information into the

call record which was started at the beginning of the call. When additional FCIs are used it is recommended to arm an EDP-R (indicating the end of call or connection configuration) to be able to apply FCI before the termination of the call record generation. The charging scenarios supported by this operation are: 2.2, 2.3 and 2.4 (refer to Appendix II/Q.1214, "Charging scenarios").

##### 3.3.23.1.1	Parameters

-	FCIBillingChargingCharacteristics:

This parameter indicates billing and/or charging characteristics. Its content is network operator specific. Depending on the applied charging scenario, the following information elements can be included (refer to Appendix II/Q.1214, "Charging scenarios"):

·	complete charging record (scenario 2.2);

·	charge party (scenario 2.3);

·	charge level (scenario 2.3);

·	charge items (scenario 2.3);

·	correlationID (scenario 2.4).

#### 3.3.23.2	Invoking Entity (SCF)

##### 3.3.23.2.1	Normal procedure

SCF Preconditions:

1)	A control relationship exists between the SCF and the SSF.

2)	An SLPI has determined that a "FurnishChargingInformation" has to be sent by the SCF.

SCF Postconditions:

1)	No FSM state transition.

2)	SLPI execution may continue.

The SCSM-FSM is in state "Preparing SSF instruction" or is in state "Queueing FSM". This operation is invoked by the SCF if a SLPI results in the request of creating a call record to the SSF or to include some billing or charging information into the default call record. In the case of call queueing, this operation may contain information pertaining to the initiation of queueing or the call queueing time duration for call logging purpose. This causes no SCSM-FSM state transition.

##### 3.3.23.2.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

#### 3.3.23.3	Responding Entity (SSF)

##### 3.3.23.3.1	Normal procedure

SSF Preconditions:

–	SSF-FSM: State c, "Waiting for Instructions"; or

SSF-FSM: State d, "Waiting for End of User Interaction"; or

SSF-FSM: State e, "Waiting for End of Temporary Connection"; or

SSF-FSM: State f, "Monitoring"; or

Assisting/hand-off SSF-FSM State b, "Waiting for Instructions".

SSF Postcondition:

–	No FSM state transition.

On receipt of this operation, the SSF performs actions to create the call record according the off line charging scenario which is applicable using the information elements included in the operation:

-	registers the complete call record included in the operation;

-	generates and registers a call record according the information (charge party, charge level, charge items);

-	include the information received "correlationID" in the default call record which is generated and, registered by default at the SSF.

By means of a parameter at the "FurnishChargingInformation" operation the SCF can initiate the pulse metering function of the SSF.

In that case the SSF shall generate meter pulses according to the applicable charging level, account and record them.

The SSF records charge related data like for example the call duration, begin time stamp or end time stamp. Additionally the SSF records further data if required.

The charging level can be determined by:

a)	the SCF; or

b)	the SSF; or

c)	a succeeding exchange; or

d)	the post processing function.

If a) applies, the charging level is included in the "FurnishChargingInformation" operation.

If b) applies, the SSF shall determine the charging level based on the corresponding parameters contained in the operation.

If c) applies, either the "FurnishChargingInformation" operation contains the corresponding parameters indicating that the charging level shall be determined in a succeeding exchange, or the SSF detects during the determination of the charging level based on the provided parameters that the charging level shall be determined in a succeeding exchange.

The SSF can either account received pulses or convert any charging messages received from the B-side to pulses. In both cases, the accumulated pulses are included when the IN call record is generated or ignored.

##### 3.3.23.3.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

### 3.3.24	HoldCallInNetwork procedureXE "HoldCallInNetwork procedure"§

#### 3.3.24.1	General description

This operation is used to provide the capability of queueing a call during the setup phase (e.g. to provide a call completion to busy, a call would be queued until the destination becomes free).

##### 3.3.24.1.1	Parameters

–	holdcause:

This parameter is optional. When used, it is network-specific and denotes the cause for holding the call.

#### 3.3.24.2	Invoking Entity (SCF)

##### 3.3.24.2.1	Normal procedure

SCF Precondition:

–	The SCSM is in state 2.2.1, "Preparing SSF Instructions", and the event (e2.2.3) Busy_Line/Trunk has taken place.

SCF Postcondition:

–	The SCSM enters state 2.2.2 Queueing.

##### 3.3.24.2.2	Error handling

There is no specific error treatment associated with this operation. Generic error handling for the operation related errors is described in 3.2 and the TCAP service which are used for reporting operation errors are described in 3.4.

#### 3.3.24.3	Responding Entity (SSF)

##### 3.3.24.3.1	Normal procedure

SSF Precondition:

–	The SSF-FSM is in state c, "Waiting for Instructions".

SSF Postcondition:

–	The SSF-FSM remains in the same state.

##### 3.3.24.3.2	Error handling

There is no operation-specific error treatment associated with this operation. Generic error handling for the operation-related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

### 3.3.25	InitialDP procedureXE "InitialDP procedure"§

#### 3.3.25.1	General description

This operation is sent by the SSF after detection of a TDP-R in the BCSM, to request the SCF for instructions to complete the call.

##### 3.3.25.1.1	Parameters

·	serviceKey:

This parameter identifies for the SCF unambiguously the requested IN service. It is used to address the correct application/SLP within the SCF (not for SCP addressing).

·	calledPartyNumber:

This parameter contains the number used to identify the called party in the forward direction, i.e. see Recommendation Q.762.

·	callingPartyNumber:

See Q.762 Calling Party Number signalling information.

·	callingPartysCategory:

See Q.762 Calling Party Category signalling information.

·	cGEncountered:

See Recommendation Q.1290.

·	iPSSPCapabilities:

See Recommendation Q.1290.

·	iPAvailable:

See Recommendation Q.1290.

·	locationNumber:

This parameter is used to convey the geographical area address for mobility services, see Recommendation Q.762. It is used when 'callingPartyNumber' does not contain any information about the geographical location of the calling party (e.g. origin dependent routing when the calling party is a mobile subscriber).

·	originalCalledPartyID:

See Q.762 Original Called Number signalling information.

·	triggerType:

This parameter indicates to the SCF the particular event which caused the detection of a valid trigger condition.

·	highlayerCompatibility:

This parameter indicates the type of the high layer compatibility, which will be used to determine the ISDN-teleservice of a connected ISDN terminal. For encoding DSS-1 (see Recommendation Q.931) is used. The highlayerCompatibility can also be transported by ISUP within the ATP (see Recommendation Q.763) parameter.

·	serviceInteractionIndicators:

This parameter contains indicators sent from the SSF to the SCF for control of the network based services at the originating exchange and the destination exchange.

·	additionalCallingPartyNumber:

The calling party number provided by the access signalling system of the calling user, e.g. provided by a PBX.

·	forwardCallIndicators:

This parameter indicates if the call shall be treated as a national or international call. It also indicates the signalling capabilities of the network access, preceding network connection and the preferred signalling capabilities of the succeeding network connection. The network access capabilities does not indicate the terminal type. For example, an ISPBX will have an ISDN type of access, but the end user terminal behind the ISPBX may be ISDN or non-ISDN.

·	bearerCapability:

This parameter indicates the type of the bearer capability connection or the transmission medium requirements to the user. It is a network option to select one of the two parameters to be used:

–	bearerCap:

This parameter contains the value of the DSS-1 Bearer Capability parameter (see Recommendation Q.931) in case the SSF is at local exchange level or the value of the ISUP User Service Information parameter (see Recommendation Q.763) in case the SSF is at transit exchange level.

The parameter 'bearerCapability' shall only be included in the 'InitialDP' operation in case the DSS-1 Bearer Capability parameter or the ISUP User Service Information parameter is available at the SSP.

If two values for bearer capability are available at the SSF or if User Service Information and User Service Information Prime are available at the SSF, the 'bearerCap' shall contain the value of the preferred bearer capability respectively the value of the User Service Information Prime parameter.

–	tmr:

The tmr is encoded as the Transmission Medium Requirement parameter of the ISUP according to Recommendation Q.763.

If two values for transmission medium requirement are available at the SSF or if Transmission Medium Requirement and Transmission Medium Requirement Prime are available at the SSF, the 'bearerCap' shall contain the value of the preferred transmission medium requirement respectively the value of the Transmission Medium Requirement Prime parameter.

·	eventTypeBCSM:

This parameter indicates the armed BCSM detection point event, resulting in the 'InitialDP' operation.

·	redirectingPartyID:

This parameter indicates the last directory number the call was redirected from.

·	redirectionInformation:

See Q.763 Redirection Information signalling information.

#### 3.3.25.2	Invoking Entity (SSF)

##### 3.3.25.2.1	Normal procedure

SSF Preconditions:

1)	An event fulfilling the criteria for the DP being executed has been detected.

2)	Call gapping and SS No. 7 overload are not in effect for the call, and the call is not to be filtered.

SSF Postconditions:

1)	A control relationship has been established if the DP was armed as a TDP-R. The SSF-FSM moves to the state "Waiting for Instructions".

2)	The SSF-FSM remains in the state "Idle" if the DP was armed as a TDP-N.

Following a trigger detection (due to the DP criteria assigned being met) related to an armed TDP in the BCSM caused by a call origination attempt, the SSF checks if call gapping, SS No. 7 overload or service filtering are not in effect for the related call segment.

If these conditions are met, then the 'InitialDP' operation is invoked by the SSF. The address of the SCF the 'InitialDP' operation has to be sent to, is determined on the base of trigger related data. The SSF provide as many parameters as available. In some cases, some parameters must be available (such as 'callingPartyNumber' or 'callingPartyCategory'). This is to be handled appropriately by the SSF in its trigger table (to know that such parameters are necessary for some triggering conditions) and in conducting the necessary action to get these parameters if they are not available (for instance if non-SS No. 7 signalling is used, it may be possible to request the 'callingPartyCategory' from a preceding exchange).

Otherwise, the call control is given back to the underlying network.

If the DP was armed as a TDP-R, a control relationship is established to the SCF. The SSF application timer TSSF is set when the SSF sends 'InitialDP' for requesting instructions from the SCF. It is used to prevent excessive call suspension time.

##### 3.3.25.2.2	Error handling

If the destination SCF is not accessible, then the call is given final treatment.

On expiration of TSSF before receiving any operation, the SSF aborts the interaction with the SCF and the call is given final treatment, e.g. routing to a final announcement.

If the calling party abandons after the sending of 'InitialDP', then the SSF aborts the control relationship by means of an abort to TCAP. Note that TCAP will wait until the first response message from the SCF has been received before it sends an abort to the SCF (see also 3.2).

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

#### 3.3.25.3	Responding Entity (SCF)

##### 3.3.25.3.1	Normal procedure

SCF Precondition:

–	None.

SCF Postcondition:

–	An SLPI has been invoked.

On receipt of 'InitialDP' operation the SCSM moves from "Idle" to the state "Preparing SSF Instructions", a control relationship to the related SSF is created. A Service Logic Program Instance (SLPI) is invoked for processing the 'InitialDP' operation based on the 'serviceKey' parameter. By means of this control relationship, the SCF may influence the Basic Call Processing in accordance with the service logic invoked.

The actions to be performed in the SLPI depend on the parameters conveyed via this operation and the SLPI, i.e. the requested IN service itself.

##### 3.3.25.3.2	Error handling

If the 'InitialDP' operation is rejected, then the SCSM remains in "Idle". The maintenance function is informed and no SLPI is invoked.

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

### 3.3.26	InitiateCallAttempt procedureXE "InitiateCallAttempt procedure"§

#### 3.3.26.1	General description

This operation is used to request the SSF to create a new call to one call party using the address information provided by the SCF (e.g. wake-up call). An EDP-R must be armed on answer and all the call failure events, in order to have the SCF, treat this call appropriately when either of these events is encountered.

##### 3.3.26.1.1	Parameters

–	destinationRoutingAddress:

This parameter contains the called party number towards which the call is to be routed. The destinationRoutingAddress may contain one called party number only for this operation. The encoding of the parameter is defined in Recommendation Q.763.

–	alertingPattern:

See Recommendation Q.1290. It only applies if the network signalling supports this parameter or if SSF is the terminating local exchange for the subscriber.

–	iSDNAccessRelatedInformation:

Carries the same information as the protocol element ISUP Access Transport parameter in Recommendation Q.762.

–	travellingClassMark:

The SCF uses the travellingClassMark parameter to provide essential route selection information to the SSF. The SSF uses this information to populate the outgoing ISUP-IAM message, the population and mapping of this parameter is network operator specific.

–	serviceInteractionIndicators:

This parameter contains indicators sent from the SCF to the SSF for control of the network based services at the originating exchange and the destination exchange.

–	callingPartyNumber:

This parameter identifies which number shall be regarded as the calling party for the created call. If this parameter is not sent by the SCF, the SSF may supply a network dependent default value.

#### 3.3.26.2	Invoking Entity (SCF)

##### 3.3.26.2.1	Normal procedure

SCF Preconditions:

1)	An SLPI has been invoked, no control relationship exists between SCF and SSF.

2)	An SLPI has determined that an 'InitiateCallAttempt' operation should be sent to the SCF.

3)	The SCSM-FSM is in state "Idle".

SCF Postconditions:

1)	A control relationship is established between the SCF and SSF.

2)	The SCSM-FSM is in state "Preparing SSF Instructions".

3)	SLPI execution continues.

The SCSM-FSM moves to state "Preparing SSF Instructions" when the service logic invokes this operation. In order to enable the establishment of a control relationship between the SCF and SSF and to allow the SCF to control the created call appropriately, the SLPI shall monitor for the BCSM event(s) which report the result of the created call set-up. This includes DP 3 or DP 4, 5, 6, and 7. Any other Non-Call_Processing_Instructions may be sent as well. The 'InitiateCallAttempt' operation creates a BCSM instance in the SSF but the SSF suspends the call processing of this BCSM. The SLPI shall send a 'Continue' operation to request the SSF to route the call to the specified destination. The SCSM-FSM shall proceed as specified at the procedure for the 'Continue' operation.

The above described procedure shall be part of the establishment of the control relationship, i.e. operations up to and including the 'Continue' operation shall be sent together in the same message to the SSF. The SCF shall start a response Timer TSCF when the 'InitiateCallAttempt' operation is sent. The response Timer shall supervise the confirmation of the dialogue from SSF, the value of TSCF shall be equal or less than the network no answer timer.

##### 3.3.26.2.2	Error handling

On expiration of TSCF the SCF shall abort the dialogue, report the abort to the maintenance functions and inform the SLPI on the failure of dialogue establishment. The SCSM-FSM moves to the "Idle" state.

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

#### 3.3.26.3	Responding Entity (SSF)

##### 3.3.26.3.1	Normal procedure

SSF Precondition:

–	None.

SSF Postconditions:

1)	A new originating BCSM has been created, call processing is suspended at DP 1.

2)	The SSF-FSM has moved from "Idle" state to state "Waiting for Instructions".

Upon reception of 'InitiateCallAttempt', the SSF creates a new originating BCSM and suspends the call processing of this BCSM at DP 1. All subsequent operations are treated according to their normal procedures.

The properties and capabilities, normally received from or associated to the calling party, required for the call set-up shall have a network dependent default value. If a calling party number is supplied by the SCF, these properties may be dependent on the received calling party number

##### 3.3.26.3.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

### 3.3.27	ModifyEntry procedureXE "ModifyEntry procedure"§

#### 3.3.27.1	General description

The X.500 'ModifyEntry' operation is used to request the SDF to make one or several modifications to a data object. The modifications concern the attributes and their values of which the object is composed. The type of modifications to perform is given in the operation argument provided by the SCF. The modifications do not concern the values of the attributes used to identify the object (i.e. the object name). For a full description of the ModifyEntry operation, see 11.3/X.511.

##### 3.3.27.1.1	Parameters

See 11.3.2/X.511 and 11.3.3/X.511.

#### 3.3.27.2	Invoking Entity (SCF)

##### 3.3.27.2.1	Normal procedure

SCF Precondition:

–	SCSM: "SDF Bound" or "Wait for Subsequent Requests".

SCF Postcondition:

–	SCSM: "SDF Bound".

When the SCSM is in the state "Wait for Subsequent Requests" and a need of the service logic to modify information of the SDF exists, an internal event [(e2) Request_to_SDF] occurs. Until the application process indicates, with a delimitor (or a timer expiry), that the operation should be sent, the SCSM remains in the state "Wait for Subsequent Requests" and the operation is not sent. The operation is sent to the SDF in a message containing a Bind argument. The SCSM waits for the response from the SDF. The reception of the response [(E5) Response_from_SDF_with_Bind or (E4) Bind_Error] to the Bind operation previously issued to the SDF causes a transition of the SCF to the state "SDF Bound" or to the state "Idle". When the SCSM has moved to state "Idle", the ModifyEntry operation was discarded. In the state "SDF Bound", the response of the ModifyEntry operation [(E7) Response_from_SDF] causes a transition of the SCF to the same state ("SDF Bound"). It may be either the result of the ModifyEntry operation or an error.

When the SCSM is in the state "SDF Bound" and a need of the service logic to modify information of the SDF exists, an internal event occurs. This event, called (e6) Request_to_SDF causes a transition to the same state "SDF Bound" and the SCSM waits for the response from the SDF. The reception of the response [(E7) Response_from_SDF] to the ModifyEntry operation previously issued to the SDF causes a transition of the SCF to the same state "SDF Bound". The response from the SDF may be either the result of the ModifyEntry operation or an error.

##### 3.3.27.2.2	Error handling

Generic error handling for the operation related errors is described in 11.3.4/X.511 and 11.3.5/X.511 and the TCAP services that are used for reporting operating errors are described in 2.2.2.

#### 3.3.27.3	Responding Entity (SDF)

##### 3.3.27.3.1	Normal procedure

SDF Precondition:

–	SDSM: "SCF Bound" or "Bind Pending".

SDF Postcondition:

–	SDSM "SCF Bound".

When the SDF is in the state "Bind Pending", the external event (E3) Request_from_SCF caused by the reception of a 'ModifyEntry' operation from the SCF occurs. The SDF does not proceed to the operation until a Bind operation has been successfully executed. It remains in the same state.

When the SDF is in the state "SCF Bound", the external event (E7) Request_from_SCF caused by the reception of a 'ModifyEntry' operation from the SCF occurs. The SDF waits for the response to the operation.

On the receipt of the event (E7) and before updating the different attributes specified in the operation parameters, the SDF shall take the following actions:

–	verify that the object accessed by the request exists;

–	verify that the user on behalf of whom the request is performed has sufficient access rights to modify the object for each elementary modifications contained in the operation;

–	verify that each attribute or value on which an operation should be performed exists in the object;

–	verify that the proposed modifications are compatible with the object class of the object or with the syntax of the attribute.

After the specified actions, indicated above, are successfully executed for all the modifications requested in the operation, all the modifications for a same attribute take place in the order given in the "changes" parameter. A result is returned to the SCF. The sending of the result corresponds to the event (e6) Response_to_SCF.

##### 3.3.27.3.2	Error handling

Generic error handling for the operation related errors is described in 11.3.4/X.511 and 11.3.5/X.511 and the TCAP services that are used for reporting operating errors are described in 2.2.2.

### 3.3.28	OAnswer procedureXE "OAnswer procedure"§

#### 3.3.28.1	General description

This operation is sent from the SSF to the SCF at the O_Answer DP, after detecting a valid trigger condition, or to report an event requested by RequestReportBCSMEvent.

##### 3.3.28.1.1	Parameters

–	dPSpecificCommonParameters:

–	callingPartyBusinessGroupID:

See Recommendation Q.1290. The SCF can use this parameter to select SLPs based on the group and for authorization purposes. The network operators can specify that this parameter should be used if their particular network has the information available.

–	callingPartySubaddress:

See Q.931 Calling Party Subaddress.

–	callingFacilityGroup:

See Recommendation Q.1290.

–	callingFacilityGroupMember:

See Recommendation Q.1290.

–	originalCalledPartyID:

See Q.762 Original Called Number signalling information.

–	redirectingPartyID:

This parameter (if available) is the directory number of the last redirecting party.

–	redirectionInformation:

See Q.763 Redirection Information signalling information.

–	routeList:

routeList represents the list of routes which would have been used in order to route the call. The network operators can specify that this parameter should be used if their particular network has the information available.

–	travellingClassMark:

As defined in Recommendation Q.1290.

#### 3.3.28.2	Invoking Entity (SSF)

##### 3.3.28.2.1	Normal procedure

SSF Preconditions (TDP and EDP):

1)	Call origination attempt has been initiated.

2)	Indication received that the call has been accepted and the terminating party has answered.

3)	Call gapping or service filtering are not in effect.

4)	DP criteria are met.

5)	For a TDP-R, there is no existing control relationship.

6)	For an EDP-R, there is an existing control relationship and the EDP O_Answer is armed.

7)	For an EDP-N, there is an existing control or monitoring relationship and the EDP O_Answer is armed.

SSF Postconditions for TDP:

1)	For a TDP-R, basic call processing has been suspended at O_Answer DP, and a control relationship has been established.

2)	For a TDP-N, basic call processing proceeds at O_Active PIC, and no control relationship has been established.

SSF Postconditions for EDP:

1)	The SSF-FSM stays in the state "Monitoring" if the message type was notification and there are still EDPs armed or a "CallInformationReport" or "ApplyChargingReport" requested.

2)	The SSF-FSM moves to the state "Idle" if the message type was notification and there are no more EDPs armed, no "CallInformationReport" or "ApplyChargingReport" are requested.

3)	The SSF-FSM moves to the state "Waiting for Instructions" if the message type was request. Call processing is interrupted.

##### 3.3.28.2.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

#### 3.3.28.3	Responding Entity (SCF)

##### 3.3.28.3.1	Normal procedure

SCF Precondition (TDP):

–	None.

SCF Preconditions (EDP):

1)	For an EDP-R at the SSF, an existing control relationship is in place and an SLPI is running.

2)	For an EDP-N, an existing monitoring relationship is in place and an SLPI is running.

SCF Postconditions (TDP):

1)	An SLPI has been invoked.

2)	For a TDP-R, a control relationship is established, and an SLPI has been invoked.

3)	For a TDP-R, an SSF instruction is being prepared.

4)	For a TDP-N, no relationship is established. An SLPI has been invoked, executes and terminates.

SCF Postconditions (EDP):

–	For an EDP, the SCSM-FSM stays in the substate "Waiting for Notification or Request" if the message type was notification and there are still EDPs armed or a "CallInformationReport" or "ApplyChargingReport" requested; or

the SCSM-FSM moves to the state "Idle" if the message type was notification and there are no more EDPs armed, no "CallInformationReport" or "ApplyChargingReport" are requested; or

the SCSM-FSM moves to the state "Preparing SSF Instructions" if the message type was request.

##### 3.3.28.3.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

### 3.3.29	ODisconnect procedureXE "ODisconnect procedure"§

#### 3.3.29.1	General description

This operation is sent by the SSF to the SCF after detecting a valid trigger condition at the O_Disconnect DP, or to report an event requested by RequestReportBCSMEvent. Refer to 4.2.2.2/Q.1214 for additional call modelling related semantics.

##### 3.3.29.1.1	Parameters

–	serviceAddressInformation (serviceKey, miscCallInfo, triggerType):

See Recommendation Q.1290. The triggerType indicates the particular event which caused call suspension. The miscCallInfo indicates that a request for instructions or a notification has been issued to the SCF and the category for the trigger (line, group, office). The serviceKey is used in identifying the service logic to be invoked.

–	bearerCapability:

This parameter indicates the type of bearer capability connection to the user. Refer to 6.4.4/Q.1214 for population rules for the bearer capability parameter.

–	calledPartyNumber:

See Q.762 Called Party Number signalling information. This parameter is used to identify the called party in the forward direction.

–	callingPartyNumber:

See Q.762 Calling Party Number signalling information. Refer to 6.4.4/Q.1214 for population rules for the callingPartynumber parameter.

–	callingPartysCategory:

See Q.762 Calling Party’s Category signalling information. Refer to 6.4.4/Q.1214 for population rules for the callingPartysCategory parameter.

–	iPSSPCapabilities:

See Recommendation Q.1290.

–	iPAvailable:

See Recommendation Q.1290.

–	iSDNAccessRelatedInformation:

This parameter contains (possibly multiple) information elements as per Recommendation Q.931. See Access Transport Parameter signalling information in Recommendations Q.762, Q.763 and Q.931. Refer to 6.4.4/Q.1214 (Analysed Information) for population rules for ISDN access related information.

–	cGEncountered:

See Recommendation Q.1290.

–	locationNumber:

See Q.762 Location Number signalling information. This parameter is used to convey the geographical area address for mobility services. It is used when the "callingPartyNumber" does not contain any information about the geographical location of the calling party (e.g. origin dependent routing when the calling party is a mobile subscriber).

–	serviceProfileIdentifier:

See Annex A/Q.932. Refer to 6.4.4/Q.1214 for population rules for serviceProfileIdentifier.

–	terminalType:

See Recommendation Q.1290. This parameter identifies the terminal type so that the SCF can specify, to the SRF, the appropriate type of capability (voice recognition, DTMF, display capability, etc.).

–	chargeNumber:

See Recommendation Q.1290. Refer to 6.4.4/Q.1214 for population rules for chargeNumber.

–	servingAreaID:

See Recommendation Q.1290.

–	callingPartyBusinessGroupID:

See Recommendation Q.1290.

–	callingPartySubaddress:

See Q.931 Calling Party Subaddress.

–	callingFacilityGroup:

See Recommendation Q.1290.

–	callingFacilityGroupMember:

See Recommendation Q.1290.

–	releaseCause:

Indicates the cause of the disconnect.

–	routeList:

See Recommendation Q.1290.

–	carrier:

See Recommendation Q.1290.

–	connectTime:

Indicates the duration between the received answer indication from the called party side and the release of the connection.

#### 3.3.29.2	Invoking Entity (SSF)

##### 3.3.29.2.1	Normal procedures

SSF Preconditions:

1)	Call origination attempt has been initiated.

2)	Indication received from Terminating BCSM that the call is accepted and the terminating party has answered.

3)	Disconnect indication received from an originating party, or received from the terminating party via the terminating BCSM.

4)	For a TDP, call gapping or service filtering are not in effect.

5)	DP criteria have been met.

6)	For a TDP-R or a TDP-N, there is no existing control relationship.

7)	For an EDP, there is an existing control relationship and the EDP O_Disconnect is armed.

SSF Postconditions:

1)	For a TDP-R, basic call processing has been suspended at O_Disconnect DP, and a control relationship has been established.

2)	For a TDP-N, call processing proceeds to the O_Null and Authorize Termination Attempt PIC, and no control relationship has been established.

3)	For an EDP, as for EventReportBCSM procedure (see 3.3.22.2.1).

##### 3.3.29.2.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP service which are used for reporting operation errors are described in 3.4.

#### 3.3.29.3	Responding Entity (SCF)

##### 3.3.29.3.1	Normal procedure

SCF Precondition (TDP):

–	None.

SCF Preconditions (EDP):

1)	For an EDP-R at the SSF, an existing control relationship is in place and an SLPI is running.

2)	For an EDP-N, an existing monitoring relationship is in place and an SLPI is running.

SCF Postconditions (TDP):

1)	An SLPI has been invoked.

2)	For a TDP-R, a control relationship is established, and an SLPI has been invoked.

3)	For a TDP-R, an SSF instruction is being prepared.

4)	For a TDP-N, no relationship is established. An SLPI has been invoked, executes and terminates.

SCF Postconditions (EDP):

–	For an EDP, the SCSM-FSM stays in the substate "Waiting for Notification or Request" if the message type was notification and there are still EDPs armed or a "CallInformationReport" or "ApplyChargingReport" requested; or

the SCSM-FSM moves to the state "Idle" if the message type was notification and there are no more EDPs armed, no "CallInformationReport" or "ApplyChargingReport" are requested; or

the SCSM-FSM moves to the state "Preparing SSF Instructions" if the message type was request.

##### 3.3.29.3.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP service which are used for reporting operation errors are described in 3.4.

### 3.3.30	ONoAnswer procedureXE "ONoAnswer procedure"§

#### 3.3.30.1	General description

This operation is sent from the SSF to the SCF at the O_No_Answer DP, after detecting a valid trigger condition, or to report an event requested by RequestReportBCSMEvent.

##### 3.3.30.1.1	Parameters

–	dPSpecificCommonParameters:

–	callingPartyBusinessGroupID:

See Recommendation Q.1290. The SCF can use this parameter to select SLPs based on the group and for authorization purposes. The network operators can specify that this parameter should be used if their particular network has the information available.

–	callingPartySubaddress:

See Q.931 Calling Party Subaddress.

–	callingFacilityGroup:

See Recommendation Q.1290.

–	callingFacilityGroupMember:

See Recommendation Q.1290.

–	originalCalledPartyID:

See Q.762 Original Called Number signalling information.

–	prefix:

As defined in Recommendation Q.1290.

–	redirectingPartyID:

This parameter (if available) is the directory number of the last redirecting party.

–	redirectionInformation:

See Q.763 Redirection Information signalling information.

–	routeList:

routeList represents the route used.

–	travellingClassMark:

As defined in Recommendation Q.1290.

–	carrier:

As defined in Recommendation Q.1290.

#### 3.3.30.2	Invoking Entity (SSF)

##### 3.3.30.2.1	Normal procedure

SSF Preconditions (TDP and EDP):

1)	Call origination attempt has been initiated.

2)	Indication received that the terminating party has not answered within the specified time period.

3)	Call gapping or service filtering are not in effect.

4)	DP criteria have been met (TDP or EDP).

5)	For a TDP-R, there is no existing control relationship.

6)	For an EDP-R, there is an existing control relationship and the EDP O_No_Answer is armed.

7)	For an EDP-N, there is an existing control or monitoring relationship and the EDP O_No_Answer is armed.

SSF Postconditions for TDP:

1)	For a TDP-R, basic call processing has been suspended at O_No_Answer DP, and a control relationship has been established.

2)	For a TDP-N, default exception handling has been provided, and no control relationship has been established. Use of TDP-N at this DP implies that there is no further alerting of the called party and that switch based no-answer treatments, as applicable, may be invoked.

SSF Postconditions for EDP:

1)	The SSF-FSM stays in the state "Monitoring" if the message type was notification and there are still EDPs armed or a 'CallInformationReport' or 'ApplyChargingReport' requested.

2)	The SSF-FSM moves to the state 'Idle' if the message type was notification and there are no more EDPs armed, no 'CallInformationReport' or 'ApplyChargingReport' are requested.

3)	The SSF-FSM moves to the state "Waiting for Instructions" if the message type was request. Call processing is interrupted.

##### 3.3.30.2.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

#### 3.3.30.3	Responding Entity (SCF)

##### 3.3.30.3.1	Normal procedure

SCF Precondition (TDP):

–	None.

SCF Preconditions (EDP):

1)	For an EDP-R at the SSF, an existing control relationship is in place and an SLPI is running.

2)	For an EDP-N, an existing monitoring relationship is in place and an SLPI is running.

SCF Postconditions (TDP):

1)	An SLPI has been invoked.

2)	For a TDP-R, a control relationship is established, and an SLPI has been invoked.

3)	For a TDP-R, an SSF instruction is being prepared.

4)	For a TDP-N, no relationship is established. An SLPI has been invoked, executes and terminates.

SCF Postconditions (EDP):

–	For an EDP, the SCSM-FSM stays in the substate "Waiting for Notification or Request" if the message type was notification and there are still EDPs armed or a 'CallInformationReport' or 'ApplyChargingReport' requested; or

The SCSM-FSM moves to the state "Idle" if the message type was notification and there are no more EDPs armed, no 'CallInformationReport' or 'ApplyChargingReport' are requested; or

The SCSM-FSM moves to the state "Preparing SSF Instructions" if the message type was request.

##### 3.3.30.3.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

### 3.3.31	OriginationAttemptAuthorized procedureXE "OriginationAttemptAuthorized procedure"§

#### 3.3.31.1	General description

This operation is sent by the SSF after Authorization for call attempt is complete from BCSM O_Null and Authorize Origination Attempt PIC processing and a TDP has been detected.

##### 3.3.31.1.1	Parameters

–	serviceAddressInformation (triggerType, serviceKey, miscCallInfo):

The triggerType indicates the particular event which caused call suspension. The miscCallInfo indicates that a request for instructions has been issued to the SCF and the category for the trigger (line, group, office). The serviceKey is used in identifying the service logic to be invoked.

–	bearerCapability:

See Q.762 User Service Information and Q.931 Bearer Capability information element. Refer to 6.4.4/Q.1214 for population rules for the bearer capability parameter.

–	calledPartyNumber:

See Q.762 Called Party Number signalling information. This parameter is used to identify the called party in the forward direction (i.e. it is used to populate the bearer signalling protocol’s Called Party Number information element).

–	callingPartyNumber:

See Q.762 Calling Party Number sigalling information. This parameter identifies the directory number of the originating line or the calling party number received in ISUP signalling. Refer to 6.4.4/Q.1214 for population rules for the callingPartyNumber parameter.

–	callingPartysCategory:

See Q.762 Calling Party’s Category signalling information. This parameter indicates the type of the calling party (e.g. operator, pay phone, ordinary subscriber). Refer to 6.4.4/Q.1214 for population rules for the callingPartysCategory parameter.

–	iPSSPCapabilities:

See Recommendation Q.1290.

–	iPAvailable:

See Recommendation Q.1290.

–	iSDNAccessRelatedInformation:

Carries the same information as the protocol element ISUP Access Transport parameter in Recommendation Q.762. This parameter contains information elements provided across an ISDN interface. Refer to 6.4.4/Q.1214 for population rules for ISDN access related information.

–	cGEncountered:

This parameter indicates that the related call has passed call gapping.

–	locationNumber:

See Q.762 Location Number signalling information. This parameter is used to convey the geographical area address for mobility services. It is used when the 'callingPartyNumber' does not contain any information about the geographical location of the calling party (e.g. origin dependent routing when the calling party is a mobile subscriber).

–	serviceProfileIdentifier:

See Annex A/Q.932. This parameter allows ISDN to support identification and selection of specific terminals on a multi-point user-network interface to support multiple user service profiles when Q.931 information elements are not sufficient. See 6.4.4/Q.1214 for population rules for serviceProfileIdentifier.

–	terminalType:

See Recommendation Q.1290. Identifies the terminal type so that the SCF can specify, to the SRF, the appropriate type of capability (voice recognition, DTMF, display capability, etc.).

–	chargeNumber:

See Recommendation Q.1290. Refer to 6.4.4/Q.1214 for population rules for chargeNumber.

–	servingAreaID:

See Recommendation Q.1290.

–	dialledDigits:

See Recommendation Q.1290. Refer to 6.4.4/Q.1214 for population rules for dialledDigits.

–	callingPartyBusinessGroupID:

See Recommendation Q.1290.

–	callingPartySubaddress:

See Q.931 Calling Party Subaddress.

–	callingFacilityGroup:

See Recommendation Q.1290.

–	callingFacilityGroupMember:

See Recommendation Q.1290.

–	travellingClassMark:

See Recommendation Q.1290. Refer to 6.4.4/Q.1214 for population rules for the travellingClassMark parameter.

–	Carrier:

See Recommendation Q.1290.

#### 3.3.31.2	Invoking Entity (SSF)

##### 3.3.31.2.1	Normal procedure

SSF Preconditions:

1)	Call processing has progressed through the O_Null and Authorize Origination Attempt PIC of the O-BCSM and the call origination attempt has been authorized.

2)	Call gapping and SS No. 7 overload are not in effect for the call, and the call is not to be filtered.

3)	A trigger is armed as a TDP at the Origination_Attempt_Authorized detection point.

SSF Postconditions:

1)	For TDP-R, a control relationship has been established and the SSF waits for instructions from the SCF.

2)	For TDP-N, call processing continues at the Collect_Info PIC.

##### 3.3.31.2.2	Error handling

If the destination SCF is not accessible, then the call is given final treatment (other treatments are for further study). On expiration of TSSF before receiving any operation, the SSF aborts the interaction with the SCF and the call is given final treatment (e.g. routing to a final announcement). If the calling party abandons after the sending of the OriginationAttemptAuthorized operation, then the SSF aborts the control relationship after the first answer message from the SCF has been received: the Transaction ID is held open until TSSF expires.

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

#### 3.3.31.3	Responding Entity (SCF)

##### 3.3.31.3.1	Normal procedure

SCF Precondition:

–	No control relationship exists.

SCF Postconditions:

1)	An SLPI has been invoked; a control relationship is established for TDP-R.

2)	For TDP-N: None.

On receipt of the OriginationAttemptAuthorized operation, the SCSM moves from "Idle" state to the state "Preparing SSF Instructions". A control relationship to the related SSF is created. A Service Logic Program Instance (SLPI) is invoked for processing the OriginationAttemptAuthorized operation. By means of this control relationship, the SCF may influence the Basic Call Processing in accordance with the service logic invoked. The actions to be performed in the SLPI depend on the parameters conveyed via this operation and the SLPI (i.e. the requested IN service itself).

##### 3.3.31.3.2	Error handling

If the OriginationAttemptAuthorized operation is rejected, then the SCSM remains in the same state. The maintenance function is informed and no SLPI is invoked. Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

### 3.3.32	PlayAnnouncement procedureXE "PlayAnnouncement procedure"§

#### 3.3.32.1	General description

This operation is used for inband interaction with an analogue user or for interaction with an ISDN user.

##### 3.3.32.1.1	Parameters

·	informationToSend:

This parameter indicates an announcement, a tone or display information to be sent to the end user by the SRF.

a)	inbandInfo:

This parameter specifies the inband information to be sent.

–	messageID:

This parameter indicates the message(s) to be sent, this can be one of the following:

i)	elementaryMessageID:

This parameter indicates a single announcement.

ii)	text:

This parameter indicates a text to be sent. The text shall be transformed to inband information (speech) by the SRF. This parameter consists of two subparameters, messageContent and attributes. The attributes of text may consist of items such as language.

iii)	elementaryMessageIDs:

This parameter specifies a sequence of announcements.

iv)	variableMessage:

This parameter specifies an announcement with one or more variable parts.

–	numberOfRepetitions:

This parameter indicates the maximum number of times the message shall be sent to the end-user.

–	duration:

This parameter indicates the maximum time duration in seconds that the message shall be played/repeated. ZERO indicates endless repetition.

–	interval:

This parameter indicates the time interval in seconds between repetitions, i.e. the time between the end of the announcement and the start of the next repetition. This parameter can only be used when the number of repetitions is > 1.

b)	tone:

This parameter specifies a tone to be sent to the end-user.

–	toneID:

This parameter indicates the tone to be sent.

–	duration:

This parameter indicates the time duration in seconds of the tone to be sent. ZERO indicates infinite duration.

c)	displayInformation:

This parameter indicates a text string to be sent to the end-user. This information can not be received by a PSTN end-user.

NOTE – As the current signalling systems (DSS-1/ISUP) do not provide an indication whether or not information can be displayed by the user’s terminal, in case of user interaction with an ISDN user two consecutive 'PlayAnnouncement' operations are sent. The first contains the display information, the second contains the inband information to be sent to the user. Since the execution of the display information by the SRF should take a limited amount of time, the inband information will be immediately sent by the SRF to the user, in sequence with the display information.

·	disconnectFromIPForbidden:

This parameter indicates whether or not the SRF should be disconnected from the user when all information has been sent.

·	requestAnnouncementComplete:

This parameter indicates whether or not a 'SpecializedResourceReport' shall be sent to the SCF when all information has been sent.

#### 3.3.32.2	Invoking Entity (SCF)

##### 3.3.32.2.1	Normal procedure

Recommendation Q.1218     (10/95)

SCF Preconditions:

1)	The SLPI detects that information should be sent to the user.

2)	A connection between the user and a SRF has been established.

3)	The SCSM-FSM is in the state "User interaction", substate "Waiting for response from the SRF".

SCF Postconditions:

1)	If "RequestAnnouncementComplete" was set TRUE, the SCSM will stay in substate "Waiting for Response from the SRF" and wait for the "SpecializedResourceReport".

2)	If "RequestAnnouncementComplete" was set FALSE and more information needs to be sent ("DisconnectFromIPForbidden" was set to TRUE), the SCSM will stay in substate "Waiting for Response from the SRF".

3)	If "RequestAnnouncementComplete" was set FALSE and no more information needs to be sent ("DisconnectFromIPForbidden" was set to FALSE), the SCSM will move to the state "Preparing SSF Instructions".

##### 3.3.32.2.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

#### 3.3.32.3	Responding Entity (SRF)

##### 3.3.32.3.1	Normal procedure

SRF Precondition:

–	The SRSM-FSM is in the state "Connected" or in the state "User Interaction" if the SRF received previously an operation from the SCF.

SRF Postconditions:

1)	The SRF sends the information to the user as indicated by "informationToSend".

2)	The SRSM-FSM moves to the state "User Interaction" or

remains in the same state.

3)	If all information has been sent and "RequestAnnouncementComplete" was set TRUE, the SRSM sends a "SpecializedResourceReport" operation to the SCF.

4)	If all information has been sent and "disconnectFromIPForbidden" was set FALSE, the SRSM disconnects the SRF from the user.

The announcement send to the end-user is ended in the following conditions:

–	if neither "duration" or "numberOfRepetitions" is specified, then the network specific announcement ending conditions shall apply; or

–	if "numberOfRepetitions" is specified, when all repetitions have been sent; or

–	if duration is specified, when the duration has expired. The announcement is repeated until this condition is met; or

–	if "duration" and "numberOfRepetitions" is specified, when one of both conditions is satisfied (whatever comes first).

##### 3.3.32.3.2	Error handling

If a Cancel operation is received before or during the processing of the operation, then the operation is immediately cancelled and the error "Cancelled" is reported to the invoking entity.

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

### 3.3.33	PromptAndCollectUserInformation procedureXE "PromptAndCollectUserInformation procedure"§

#### 3.3.33.1	General description

This operation is used to interact with a call party in order to collect information.

##### 3.3.33.1.1	Parameters

a)	collectedInfo

i)	collectedDigits

·	minimumNbOfDigits:

If this parameter is missing, the default value is defined to be 1. The "minimumNbOfDigits" specifies the minimum number of valid digits to be collected.

·	maximumNbOfDigits:

This parameter should always be present and specifies the maximum number of valid digits to be collected. The following applies: "maximumNbOfDigits" >= "minimumNbOfDigits".

·	endOfReplyDigit:

This parameter indicates the digit used to signal the end of input.

In case the "maximumNbOfDigits" = "minimumNbOfDigits", the "endOfReplyDigit" (could be present but) has no further meaning. This parameter can be one or two digits.

In case the "maximumNbOfDigits" > "minimumNbOfDigits", the following applies:

–	If "endOfReplyDigit" is not present, the end of input is indicated:

1)	when the inter-digit timer expires; or

2)	when the number of valid digits received equals the "maximumNbOfDigits".

–	If "endOfReplyDigit" is present, the end of input is indicated:

1)	when the inter-digit timer expires; or

2)	when the end of reply digit is received; or

3)	when the number of valid digits received equals the "maximumNbOfDigits".

When the end of input is attained, the collected digits are send from SRF to the SCF, including the "endOfReplyDigit" if received by the SRF.

In the case the number of valid digits received is less than the "minimumNbOfDigits" when the inter-digit timer expires or when the end of reply digit is received, the input is specified as being erroneous.

·	cancelDigit:

If this parameter is present, the cancel digit can be entered by the user to request a possible retry. All digits already received by the SRF are discarded and the same "PromptAndCollectInformation" procedure is performed again, thus, e.g. the same announcement to request user information is given to the user and information is collected. This parameter can be one or two digits .

If this parameter is not present, the user is not able to request a possible retry.

·	startDigit:

If this parameter is present, the start digit indicates the start of the valid digits to be collected. The digits that are received by the SRF before this start digit is received, are discarded and are not considered to be valid. This parameter can be one or two digits.

If this parameter is not present, all received digits are considered to be valid.

·	firstDigitTimeout:

If this parameter is present, the first digit should be received by the SRF before the first-digit timer expiration. In case the first digit is not received before first-digit timer expiration, the input is regarded to be erroneous. After receipt of the first valid or non-valid input digit, the corresponding first-digit timer is stopped.

If this parameter is not present, then the SRF uses a default value (network operator specific) for the first-digit timer in which the first valid or non-valid input digit is received.

If "startDigit" is present, the first-digit timer is stopped after the start digit is received.

·	interDigitTimeOut:

If this parameter is present any subsequent valid or non-valid digit should be received by the SRF before the inter-digit timer expires. As result the inter-digit timer is reset and restarted.

In case a subsequent valid or non-valid digit is not received before the inter-digit timer expires and the number of received valid digits is less than the "minimumNbOfDigits", the input is regarded to be unsuccessful.

In case a subsequent valid or non-valid digit is not received before the inter-digit timer expires and the number of received valid digits is greater than the "minimumNbOfDigits", and less than or equal to the "maximumNbOfDigits", the input is regarded to be successful.

If the "interDigitTimeOut" is not present, then the SRF uses a default value for the inter-digit time.

·	errorTreatment:

This optional parameter defines what specific action should be taken by the SRF in the event of error conditions occurring. The default value is reportErrorToSCF.

·	interruptableAnnInd:

This parameter is optional, where the default value is specified being TRUE.

If this parameter is TRUE, the announcement is interrupted after the first valid or non-valid digit is received by the SRF. If the announcement is interrupted, a possible start-digit timer will not apply anymore. However, if the announcement has not been interrupted, a possible start-digit timer is started after the announcement has been finished.

If this parameter is present and explicitly set to FALSE, the announcement will not be interrupted after the first digit is received by the SRF. The received digits during the announcement are discarded and considered to be non-valid. All other specified parameters ("minimumNbOfDigits", "maximumNbOfDigits", "endOfReplyDigit", etc.) do not apply before the announcement has been finished. The possible start-digit timer is started after the announcement has been finished.

·	voiceInformation:

This parameter is optional, where the default value is specified being FALSE. If the "voiceInformation" parameter is FALSE, all valid or non-valid digits are entered by DTMF.

If this parameter is present and explicitly set to TRUE, calling user is required to provide all valid or non-valid information by speech. The SRF will perform voice recognition and translation of the provided information into digits. A possible end of reply digit will also have to be provided by speech.

·	voiceBack:

This parameter is optional, where the default value is specified being FALSE. If the "voiceBack" parameter is FALSE, no voice back information is given by the SRF.

If this parameter is present and explicitly set to TRUE, the valid input digits received by the SRF will be announced back to the calling user immediately after the end of input is received. The non-valid input digits will not be announced back to the calling user.

A possible end of reply digit is not voiced back.

b)	disconnectFromIPForbidden:

This parameter indicates whether the SRF should initiate disconnection to the SSF-CCF after the interaction has been completed. If the parameter is not present or set to TRUE, the SRF shall not initiate disconnection.

c)	informationToSend:

This parameter indicates an announcement, a tone or display information to be sent to the end user by the SRF.

i)	inbandInfo:

This parameter specifies the inband information to be sent.

·	messageID:

This parameter indicates the message(s) to be sent, this can be one of the following:

–	elementaryMessageID:

This parameter indicates a single announcement.

–	text:

This parameter indicates a text to be sent. The text shall be transformed to inband information (speech) by the SRF. This parameter consist of two subparameters, messageContent and attributes. The attributes of text may consist of items such as language.

–	elementaryMessageIDs:

This parameter specifies a sequence of announcements.

–	variableMessage:

This parameter specifies an announcement with one or more variable parts.

·	numberOfRepetitions:

This parameter indicates the maximum number of times the message shall be sent to the end-user.

·	duration:

This parameter indicates the maximum time duration in seconds that the message shall be played/repeated. ZERO indicates endless repetition.

·	interval:

This parameter indicates the time interval in seconds between repetitions, i.e. the time between the end of the announcement and the start of the next repetition. This parameter can only be used when the number of repetitions is > 1.

ii)	tone:

This parameter specifies a tone to be sent to the end-user.

·	toneID:

This parameter indicates the tone to be sent.

·	duration:

This parameter indicates the time duration in seconds of the tone to be sent. ZERO indicates infinite duration.

iii)	displayInformation:

This parameter indicates a text string to be sent to the end-user. This information can not be received by a PSTN end-user.

NOTE – As the current signalling systems (DSS 1/ISUP) do not provide an indication whether or not information can be displayed by the user’s terminal, in case of user interaction with an ISDN user, the 'displayInformation' parameter is not used in the 'PromptAndCollectUserInformation' operation. Instead, a 'PlayAnnouncement' operation containing the 'displayInformation' parameter followed by a 'PromptAndCollectUserInformation' operation containing inband information are sent to the user. Since the execution of the displayed information by the SRF should take a limited amount of time, the inband information will be immediately sent after by the SRF to the user, in sequence with the displayed information.

–	digitsResponse:

This parameter contains the information collected from the end-user.

#### 3.3.33.2	Invoking Entity (SCF)

##### 3.3.33.2.1	Normal procedure

SCF Preconditions:

1)	The SLPI detects that information should be collected from the end-user.

2)	A connection between the end-user and a SRF has been established.

3)	The SCSM-FSM is in state "User Interaction", substate "Waiting for Response from the SRF".

SCF Postconditions:

1)	The collected information is received from the SRF as response to the 'PromptAndCollectUserInformation' operation.

2)	If the 'disconnectFromIPForbidden' was set to FALSE, the SCSM-FSM will move to the state "Preparing SSF Instructions".

3)	Otherwise the SCSM-FSM remains in the same state.

The SLPI may continue execution before the response is received from the 'PromptAndCollectUserInformation' operation, more than one operation may be sent to the SRF before the response is received. The 'disconnectFromIPForbidden' parameter may only be set to FALSE if the 'PromptAndCollectUserInformation' operation is the last operation sent to the SRF.

##### 3.3.33.2.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

#### 3.3.33.3	Responding Entity (SRF)

##### 3.3.33.3.1	Normal procedure

SRF Precondition:

–	The SRSM-FSM is in the state "Connected", or in state "User Interaction" if the SRF received previously an operation from the SCF.

SRF Postconditions:

1)	The SRF has sent the information to the end-user as indicated by "informationToSend".

2)	The collected information from the end-user is sent to the SCF as RETURN RESULT of the "PromptAndCollectUserInformation".

3)	If the "disconnectFromIPForbidden" was set to FALSE, the SRF initiates a bearer channel disconnect to the SSF and the SRSM-FSM moves to the state "Idle".

4)	Otherwise the SRSM-FSM moves to the state "User Interaction", or remains in the same state.

The announcement send to the end-user is ended in the following conditions:

–	if neither "duration" or "numberOfRepetitions" is specified, then the network specific announcement ending conditions shall apply; or

–	if "numberOfRepetitions" is specified, when all repetitions have been sent; or

–	if duration is specified, when the duration has expired. The announcement is repeated until this condition is met; or

–	if "duration" and "numberOfRepetitions" is specified, when one of both conditions is satisfied (whatever comes first).

The above conditions are overruled if the parameter "interruptableAnnInd" is not set to FALSE and the end-user has responded with a digit during the sending of the announcement. In this case the announcement is ended immediately. The above procedures apply only to inband information and tones send to the end-user, for "displayInformation" the end conditions are met upon sending, i.e. no interruption can occur.

The parameter "errorTreatment" specifies how the SRF shall treat the error. The default value "reportErrorToSCF" means that the error shall be reported to SCF by means of Return Error with "ImproperCallerResponse". The value "help" indicates that no error shall be reported to SCF but assistance shall be given to the end-user in form of a network dependent default announcement (which may be dependent on the context, i.e. the send message). The value "repeatPrompt" indicates that no error shall be reported to the SCF but the prompt shall be repeated to the end-user. The last two procedures shall only be done once per "PromptAndCollectUserInformation" operation.

NOTE – Processing "endOfInput": The receipt of any "endOfInput" condition (e.g. endOfReplyDigit, cancelDigit, firstDigitTimeout, interDigitTimeout) terminates immediately the ongoing input. In other words, when e.g. an endOfReplyDigit is received, the receipt of a subsequent cancelDigit will not be processed anymore.

##### 3.3.33.3.2	Error handling

If a Cancel operation is received before or during the processing of the operation, then the operation is immediately cancelled and the error "Cancelled" is reported to the invoking entity.

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

If any of the parameter restrictions are violated (e.g. minimumNbOfDigits > maximumNbOfDigits), then an operation error has occurred.

### 3.3.34	ReleaseCall procedureXE "ReleaseCall procedure"§

#### 3.3.34.1	General description

This operation is used to tear down by the SCF an existing call at any phase of the call for all parties involved in the call. This operation may not be sent to an assisting SSF, except in the case of hand-off procedure.

##### 3.3.34.1.1	Parameters

–	Cause:

A number giving an indication to the SSF about the reason of releasing this specific call. This may be used by SSF for generating specific tones to the different parties in the call or to fill in the 'cause' in the release message.

#### 3.3.34.2	Invoking Entity (SCF)

##### 3.3.34.2.1	Normal procedure

SCF Precondition:

–	State 2.1, "Preparing SSF instructions" or State 2.3, "Waiting for Notification or Request".

SCF Postcondition:

–	State 1, "Idle", if neither 'CallInformationReport' nor 'ApplyChargingReport' has to be received from the SSF. All resources (e.g. queue) related to the call are released by the SCF; or

State 2.3, "Waiting for Notification or Request" if a 'CallInformationReport' or 'ApplyChargingReport' still has to be received from the SSF.

##### 3.3.34.2.2	Error handling

Operation related error handling is not applicable, due to class 4 operation.

#### 3.3.34.3	Responding Entity (SSF)

##### 3.3.34.3.1	Normal procedure

SSF Precondition:

–	State C," Waiting for Instructions"; or

State F, "Monitoring" or hand-off waiting for instructions.

SSF Postcondition:

–	"Idle", state a, after sending any outstanding 'CallInformationReport'. Possible armed EDPs are ignored. All connections and resources related to the call are released.

##### 3.3.34.3.2	Error handling

Operation related error handling is not applicable, due to class 4 operation.

### 3.3.35	RemoveEntry procedureXE "RemoveEntry procedure"§

#### 3.3.35.1	General description

The X.500 'RemoveEntry' operation is used to request the SDF to remove a leaf entry (either an object entry or an alias entry) from the DIT. For a full description of the RemoveEntry operation, see 11.2/X511.

##### 3.3.35.1.1	Parameters

See 11.2.2/X.511 and 11.2.3/X.511.

#### 3.3.35.2	Invoking Entity (SCF)

##### 3.3.35.2.1	Normal procedure

SCF Precondition:

–	SCSM: "SDF Bound" or "Wait for Subsequent Requests".

SCF Postcondition:

–	SCSM: "SDF Bound".

When the SCSM is in the state "Wait for Subsequent Requests" and a need of the service logic to remove an entry from the SDF exists, an internal event [(e2) Request_to_SDF] occurs. Until the application process has not indicated with a delimitor (or a timer expiry) that the operation should be sent, the SCSM remains in the state "Wait for Subsequent Requests" and the operation is not sent. The operation is sent to the SDF in a message containing a Bind argument. The SCSM waits for the response from the SDF. The reception of the response [(E5) Response_from_SDF_with_Bind or (E4) Bind_Error] to the Bind operation previously issued to the SDF causes a transition of the SCF to the state "SDF Bound" or to the state "Idle". When the SCSM has moved to state "Idle", the RemoveEntry operation was discarded. In the State "SDF Bound", the response of the RemoveEntry operation [(E7) Response_from_SDF] causes a transition of the SCF to the same state ("SDF Bound"). It may be either the result of the RemoveEntry operation or an error.

When the SCSM is in the state "SDF Bound" and a need of the service logic to remove an entry from the SDF exists, an internal event occurs. This event, called (e6) Request_to_SDF causes a transition to the same state "SDF Bound" and the SCSM waits for the response from the SDF. The reception of the response [(E7) Response_from_SDF] to the RemoveEntry operation previously issued to the SDF causes a transition of the SCF to the same state "SDF Bound". The response from the SDF may be either the result of the RemoveEntry operation or an error.

##### 3.3.35.2.2	Error handling

Generic error handling for the operation related errors is described in 11.2.4/X.511 and 11.2.5/X.511 and the TCAP services that are used for reporting operating errors are described in 2.2.2.

#### 3.3.35.3	Responding Entity (SDF)

##### 3.3.35.3.1	Normal procedure

SDF Precondition:

–	SDSM: "SCF Bound"or "Bind Pending".

SDF Postcondition:

–	SDSM "SCF Bound".

When the SDF is in the state "Bind Pending", the external event (E3) Request_from_SCF caused by the reception of a 'RemoveEntry' operation from the SCF occurs. The SDF does not proceed to the operation until a Bind operation has been successfully executed. It remains in the same state.

When the SDF is in the state "SCF Bound", the external event (E7) Request_from_SCF caused by the reception of a 'RemoveEntry' operation from the SCF occurs. The SDF waits for the response to the operation.

On the receipt of the event (E7) and before removing the entry item, the SDF takes the following actions:

–	verify that the object to be removed exists and is a leaf entry;

–	verify that the access rights to remove the entry are sufficient.

After the specified actions indicated above are successfully executed, the entry is removed from the SDF database. A null result is returned to the SCF. The sending of the result corresponds to the event (e6) Response_to_SCF.

##### 3.3.35.3.2	Error handling

Generic error handling for the operation related errors is described in 11.2.4/X.511 and 11.2.5/X.511 and the TCAP services that are used for reporting operating errors are described in 2.2.2.

### 3.3.36	RequestCurrentStatusReport procedureXE "RequestCurrentStatusReport procedure"§

#### 3.3.36.1	General description

This operation is used to request the SSF to watch the current status (busy or idle) of particular termination resource.

##### 3.3.36.1.1	Parameters

–	resourceID:

This parameter indicates the physical termination resource which is requested by the SCF to be monitored by the SSF. This parameter is one of the lineID, facilityGroupID, facilityGroupMemberID, or trunkGroupID.

–	resourceStatus:

#### 3.3.36.2	Invoking Entity (SCF)

##### 3.3.36.2.1	Normal procedure

SCF Preconditions:

1)	The SLPI has been determined that a "Request Current Status Report" operation has to be sent.

2)	The SCME is in the state "Status Report Idle".

SCF Postcondition:

–	The SCME is in the state "Waiting for SSF Response Status Report".

When the SPLI requests to watch the status of physical termination resource, the SCF sends "Request Current Status Report" operation to the SSF to request the SSF watch the status of a specific termination resource. Then the SCME is moved to the state "Waiting for SSF Response Status Report" from the state "Status Report Idle". The SCME is returned to the State "Status Report Idle", when the SCF receives a response from the SSF.

Clarification on the use of this operation within or outside the context of a call is for further study.

##### 3.3.36.2.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services used for reporting operation errors are described in 3.4.

#### 3.3.36.3	Responding Entity (SSF)

##### 3.3.36.3.1	Normal procedure

SSF Precondition:

–	The SSME is in one of the following states:

state ma: "IdleManagement";

state mb: "Non_Call_Associated_Treatment".

SSF PostCondition:

–	The SSME is in the state "Non_Call_Associated_Treatment".

On receipt of this operation, the SSF watches the status of specific termination resource, and the SSF sends ReturnResult of this operation with result of the watched status to the SCF. If an error is occurred (e.g. the SSF cannot find specific termination resource), then the SSF sends ReturnError of this operation with proper error type to the SCF.

##### 3.3.36.3.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services used for reporting operation errors are described in 3.4.

### 3.3.37	RequestFirstStatusMatchReport procedureXE "RequestFirstStatusMatchReport procedure"§

NOTE – For RequestEveryStatusChangeReport procedure, see 3.3.55.

#### 3.3.37.1	General description

This operation is used to request the SSF start to monitor the status of particular termination resource is changed in the specific status (busy or idle).

##### 3.3.37.1.1	Parameters

–	resourceID:

This parameter indicates the physical termination resource which is requested by the SCF to be monitored by the SSF. This parameter is one of the lineID, facilityGroupID, facilityGroupMemberID, or trunkGroupID.

–	resourceStatus:

This parameter indicates the status which is requested to detect by SCF of termination resource.

–	correlationID:

Used by the SCF for correlation with a previous operation.

–	monitorDuration:

This parameter indicates the maximum time duration for monitoring in the SSF.

#### 3.3.37.2	Invoking Entity (SCF)

##### 3.3.37.2.1	Normal procedure

SCF Preconditions:

1)	The SLPI has been determined that a "Request First Status Match Report" operation has to be sent.

2)	The SCME is in the state "Status Report Idle".

SCF Postcondition:

–	The SCME is in the state "Waiting for SSF Response Status Report".

When the SPLI requests to monitor the status of physical termination resource, the SCF sends "Request First Status Match Report" operation to the SSF to request the SSF start to monitor a specific termination resource. Then the SCME is moved to the state "Waiting for SSF Response Status Report" from the state "Status Report Idle". After this, in the case that the SCF receives ReturnResult of this operation, the SCME is remained in same status. When the SCF receives ReturnError of this operation, the SCME is returned to the state "Status Report Idle".

The SCME is returned to the state "Status Report Idle", when the SCF is received a Status Report operation from the SSF.

Clarification on the use of this operation within or outside the context of a call is for further study.

##### 3.3.37.2.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services used for reporting operation errors are described in 3.4.

#### 3.3.37.3	Responding Entity (SSF)

##### 3.3.37.3.1	Normal procedure

SSF Precondition:

–	The SSME is in one of the following states:

state ma: "IdleManagement";

state mb: "Non_Call_Associated_Treatment".

SSF Postcondition:

–	The SSME is in one of the following states:

state ma: "IdleManagement";

state mb: "Non_Call_Associated_Treatment".

On receipt of this operation, the SSF starts to monitor the status of particular termination resource and the SSF sends ReturnResult of this operation to the SCF. If an error is occurred (e.g. the SSF cannot find specific termination resource), then the SSF sends ReturnError of this operation with proper error type to the SCF.

The SSF continuously monitors the status of particular termination resource till the status is changed in specific state or timer which is specified by monitorDuration parameter is expired. If the SSF finds the change of status in specific state, the SSF sends "Status Report" operation to the SCF with monitorCondition parameter set to "statusReport". If the timer expiration is occurred, the SSF sends "StatusReport" operation to the SCF with monitorCondition parameter is set to "timerExpired". Another case, the SSF receives "Cancel Status Report" operation from the SCF, the SSF sends "Status Report" operation to the SCF with monitorCondition parameter set to "Canceled". After the SSF sends this operation, the SSME should move to the state "IdleManagement" unless there are another processes of Non-Call Associated operation, in which case the SSF should remain in the "Non_Call_Associated_Treatment".

##### 3.3.37.3.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services used for reporting operation errors are described in 3.4.

### 3.3.38	RequestNotificationChargingEvent procedureXE "RequestNotificationChargingEvent procedure"§

#### 3.3.38.1	General description

This operation is used to instruct the SSF how to manage the charging events which are received from other FE’s not under the control of the service logic instance. The operation supports the options to cope with the interactions concerning the charging (refer to Appendix II.4/Q.1214 "Charging scenarios"). As several connection configurations may be established during a call, a possibility exists for the RequestNotificationChargingEvent (RNC) operation to be invoked on multiple occasions. For each connection configuration an RNC may be used several times.

##### 3.3.38.1.1	Parameters

·	Sequence of ChargingEvent:

This parameter contains a list of the charging events and the corresponding monitor types and corresponding legs. For each element in the list the following information elements are included:

–	eventTypeCharging:

This subparameter indicates the charging event type. Its content is network operator specific, which may be "charge pulses" or "charge messages".

–	monitorMode:

This subparameter indicates the monitorMode applicable for the corresponding "eventTypeCharging" subparameter. Monitor may be "interrupted", "notifyAndContinue" or "transparent".

–	legID:

This subparameter indicates the leg id of the corresponding event type charging subparameter.

#### 3.3.38.2	Invoking Entity (SCF)

##### 3.3.38.2.1	Normal procedure

SCF Preconditions:

1)	A control relationship exists between the SCF and the SSF.

2)	An SLPI has determined that a "RequestNotificationChargingEvent" has to be sent by the SCF.

SCF Postconditions:

1)	No FSM state transition;

2)	SLPI execution may continue.

The SCSM-FSM is in state "Preparing SSF Instruction" or is in state "Queueing FSM". This operation is invoked by the SCF if a SLPI results in the instruction of SSF how to cope with the interactions concerning the charging. This causes no SCSM-FSM state transition.

##### 3.3.38.2.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

#### 3.3.38.3	Responding Entity (SSF)

##### 3.3.38.3.1	Normal procedure

SSF Preconditions:

–	SSF-FSM: State c: "Waiting for Instructions"; or

SSF-FSM State d: "Waiting for End of User Interaction"; or

SSF-FSM State e: "Waiting for End of Temporary Connection"; or

SSF-FSM State f: "Monitoring"; or

Assisting/Hand-off SSF-FSM State b: "Waiting for Instructions".

SSF Postcondition:

–	No FSM state transition.

On receipt of this operation the SSF performs actions to cope with the interactions concerning the charging according to the information elements included in the operation. The requested charging event can be caused by:

a)	another SLPI; or

b)	another exchange.

Irrespective of by what the charging event is caused, the SSF performs one of the following actions on occurrence of the charging event (according the corresponding monitorMode):

–	Interrupted:

Notify the SCF of the charging event using "EventNotificationCharging" operation, do not process the event or propagate the signal. However, call and existing charging processing will not be suspended in the SSF.

–	NotifyAndContinue:

Notify the SCF of the charging event using "EventNotificationCharging", and continue processing the event or signal without waiting for SCF instructions (handled like EDP-N for BCSM events).

–	Transparent:

Do not notify the SCF of the event. This ends the monitoring of a previously requested charging event.

Requested charging events are monitored until ended by a transparent monitor mode (or in the case of charging events) until the end of the connection configuration.

In the case that multiple "RequestNotificationChargingEvent" operations are received for the same connection configuration with the same "eventTypeCharging" and "legID", only the last received "monitorMode" will apply.

##### 3.3.38.3.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

### 3.3.39	RequestReportBCSMEvent procedureXE "RequestReportBCSMEvent procedure"§

#### 3.3.39.1	General description

This operation is used to request the SSF to monitor for a call-related event (e.g. BCSM events such as busy or no answer), then send a notification back to the SCF when the event is detected.

NOTE – If the RequestReportBCSMEvent requests arming of the current DP from which the call processing was suspended, the next occurrence of the DP encountered during BCSM processing will be detected (i.e. not the current one from which the call was suspended).

##### 3.3.39.1.1	Parameters

·	bcsmEvents:

This parameter specifies the event or events of which a report is requested.

a)	eventTypeBCSM:

This parameter specifies the type of event of which a report is requested. Values origAttemptAuthorized and termAttemptAuthorized are not valid for the eventTypeBCSM parameter.

b)	monitorMode:

This parameter indicates how the event should be reported. When the 'monitorMode' is 'interrupted', the event shall be reported as a request, if the 'monitorMode' is 'notifyAndContinue', the event shall be reported as a notification, if the 'monitorMode' is 'transparent', the event shall not be reported.

c)	legID:

This parameter indicates the party in the call for which the event shall be reported. SCF will use the option 'sendingSideID' only.

–	sendingSideID:

The following values for 'legID' are assumed:

legID = 1 indicates the party that was present at the moment of the 'InitialDP' or a DP-specific operation (in case of a mid call trigger, the party causing the trigger), or the party that was created with an 'InitiateCallAttempt' operation.

legID = 2 indicates the party that was created with a 'Connect' operation, or in case of a mid call trigger, the party not causing the trigger.

If not included, the following defaults are assumed:

legID = 1 for the events CollectedInfo, AnalysedInformation, O-Abandonand T-Abandon.

legID = 2 for the events RouteSelectFailure, O-CalledPartyBusy, O-NoAnswer, O-Answer, T-Busy, T-NoAnswer and T-Answer.

The 'legID' parameter shall always be included for the events O-MidCall, O-Disconnect, T-MidCall and T-Disconnect.

d)	dPSpecificCriteria:

This parameter indicates information specific to the EDP to be armed.

–	numberOfDigits:

This parameter indicates the number of digits to be collected by the SSF for the CollectedInfo event. If the indicated number of digits is collected, SSF reports the event to the SCF.

–	applicationTimer:

This parameter indicates the application timer for the NoAnswer event. If the user does not answer the call within the allotted time, the SSF reports the event to the SCF. This timer is expected to be shorter than the network no-answer timer.

e)	bcsmEventCorrelationID:

Used by the SCF for correlation with a previous operation.

#### 3.3.39.2	Invoking Entity (SCF)

##### 3.3.39.2.1	Normal procedure

SCF Preconditions:

1)	A control relationship exists between the SCF and the SSF.

2)	The SLPI has decided that a request for an event report BCSM is needed.

3)	The SCSM-FSM is in the appropriate state to send 'RequestReportBCSMEvent'.

SCF Postconditions:

1)	The SCSM-FSM remains in the same state.

2)	SLPI execution continues.

3)	If all EDPs have been disarmed and neither a CallInformationReport nor an ApplyChargingReport is pending, the controlrelationship with the concerned SSF is ended. If no other relationship persists, the SCSM shall return to "Idle" state.

##### 3.3.39.2.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

#### 3.3.39.3	Responding Entity (SSF)

##### 3.3.39.3.1	Normal procedure

SSF Precondition:

–	The SSF-FSM is in either the state "Waiting for Instructions" or the state "Monitoring".

SSF Postconditions:

1)	The requested EDPs have been armed as indicated.

2)	Previously requested events are monitored until ended by a transparent monitor mode, until the end of the call, until the EDPs are detected or until the corresponding leg is released.

3)	The SSF-FSM remains in the same state.

4)	If all EDPs have been disarmed and no "CallInformationReport" or "ApplyChargingReport" has been requested, the SSF-FSM moves to the state "Idle".

##### 3.3.39.3.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

### 3.3.40	ResetTimer procedureXE "ResetTimer procedure"§

#### 3.3.40.1	General description

This class 2 operation is used by the SCF to refresh the TSSF application timer, in order to avoid the TSSF time-out at the SSF.

##### 3.3.40.1.1	Parameters

–	timerID:

This parameter has a default value identifying the TSSF timer.

–	timerValue:

This parameter specified the value to which the TSSF is to be set.

#### 3.3.40.2	Invoking Entity (SCF)

##### 3.3.40.2.1	Normal procedure

SCF Preconditions:

1)	A control relationship exists between the SCF and the SSF.

2)	An SLPI has determined by the TSCF-SSF guard timer expiration that the 'ResetTimer' operation has to be sent in order to avoid TSSF time-out at the SSF.

SCF Postcondition:

–	The SLPI reset the TSCF-SSF guard timer.

##### 3.3.40.2.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

#### 3.3.40.3	Responding Entity (SSF)

##### 3.3.40.3.1	Normal procedure

SSF Preconditions:

1)	Call origination attempt has been initiated.

2)	Basic call processing has been suspended at a DP.

3)	The SSF-FSM is in the "Waiting for Instruction" state or in the "Waiting for End of User Interaction" state or in the "Waiting for End of Temporary Connection" state or Assisting/hand-off waiting for instructions or Assisting/hand-off waiting for end of user interaction.

NOTE – Whether the TSSF is used or not in the state "Waiting for End of User Interaction" or in the state "Waiting for End of Temporary Connection" is network operator dependent.

SSF Postconditions:

1)	The TSSF timer has been reset.

2)	The SSF-FSM remains in the same state.

##### 3.3.40.3.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

### 3.3.41	RouteSelectFailure ProcedureXE "RouteSelectFailure Procedure"§

#### 3.3.41.1	General description

This operation is sent by the SSF to the SCF after detecting a valid trigger condition at the RouteSelectFailure DP, or to report an event requested by RequestReportBCSMEvent. Refer to 4.2.2.2/Q.1214 for additional call modelling related semantics.

##### 3.3.41.1.1	Parameters

–	serviceAddressInformation (serviceKey, miscCallInfo, triggerType):

See Recommendation Q.1290. The triggerType indicates the particular event which caused call suspension. The miscCallInfo indicates that a request for instructions or a notification has been issued to the SCF and the category for the trigger (line, group, office). The serviceKey is used in identifying the service logic to be invoked.

–	bearerCapability:

This parameter indicates the type of bearer capability connection to the user. Refer to 6.4.4/Q.1214 for population rules for the bearer capability parameter.

–	calledPartyNumber:

See Q.762 Called Party Number signalling information. This parameter is used to identify the called party in the forward direction.

–	callingPartyNumber:

See Q.762 Calling Party Number signalling information. Refer to 6.4.4/Q.1214 for population rules for the callingPartyNumber parameter.

–	callingPartysCategory:

See Q.762 Calling Party’s Category signalling information. Refer to 6.4.4/Q.1214 for population rules for the callingPartysCategory parameter.

–	iPSSPCapabilities:

See Recommendation Q.1290.

–	iPAvailable:

See Recommendation Q.1290.

–	iSDNAccessRelatedInformation:

This parameter contains (possibly multiple) information elements as per Recommendation Q.931. See Access Transport Parameter signalling information in Recommendations Q.762, Q.763 and Q.931. Refer to 6.4.4/Q.1214 (Analysed Information) for population rules for ISDN access related information.

–	cGEncountered:

See Recommendation Q.1290.

–	locationNumber:

See Q.762 Location Number signalling information. This parameter is used to convey the geographical area address for mobility services. It is used when the "callingPartyNumber" does not contain any information about the geographical location of the calling party (e.g. origin dependent routing when the calling party is a mobile subscriber).

–	serviceProfileIdentifier:

See Annex A/Q.932. Refer to 6.4.4/Q.1214 for population rules for serviceProfileIdentifier.

–	terminalType:

See Recommendation Q.1290. Identifies the terminal type so that the SCF can specify, to the SRF, the appropriate type of capability (voice recognition, DTMF, display capability, etc.).

–	chargeNumber:

See Recommendation Q.1290.

–	servingAreaID:

See Recommendation Q.1290.

–	dialledDigits:

See Recommendation Q.1290. Refer to 6.4.4/Q.1214 for population rules.

–	callingPartyBusinessGroupID:

See Recommendation Q.1290.

–	callingPartySubaddress:

See Recommendation Q.931.

–	callingFacilityGroup:

See Recommendation Q.1290.

–	callingFacilityGroupMember:

See Recommendation Q.1290.

–	failureCause:

See Recommendation Q.1290. Refer to 6.4.4/Q.1214 for population rules.

–	originalCalledPartyID:

See Q.762 Original Called Number signalling information.

–	prefix:

See Recommendation Q.1290. Refer to 6.4.4/Q.1214 for population rules.

–	redirectingPartyID:

This parameter (if available) is the directory number of the last redirecting party.

–	redirectionInformation:

See Q.763 Redirection Information signalling information.

–	routeList:

See Recommendation Q.1290.

–	travellingClassMark:

See Recommendation Q.1290.

–	carrier:

See Recommendation Q.1290.

#### 3.3.41.2	Invoking Entity (SSF)

##### 3.3.41.2.1	Normal procedure

SSF Preconditions:

1)	Call origination attempt has been initiated.

2)	Called Party Number is available and nature of address determined.

3)	Call gapping or service filtering are not in effect for the call segment.

4)	DP criteria have been met.

5)	For a TDP-R, there is no existing control relationship influencing the call segment.

SSF Postconditions:

1)	For a TDP-R, basic call processing has been suspended at Route_Select_Failure DP, and a control relationship has been established.

2)	For a TDP-N, basic call processing proceeds at O_Exception, and no control relationship has been established.

3)	For an EDP, as for EventReportBCSM procedure (see 3.3.22.2.1).

##### 3.3.41.2.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP service which are used for reporting operation errors are described in 3.4.

#### 3.3.41.3	Responding Entity (SCF)

##### 3.3.41.3.1	Normal procedure

SCF Preconditions (TDP):

–	None.

SCF Preconditions (EDP):

1)	For an EDP-R at the SSF, an existing control relationship is in place and an SLPI is running.

2)	For an EDP-N, an existing monitoring relationship is in place and an SLPI is running.

SCF Postconditions (TDP):

1)	An SLPI has been invoked.

2)	For a TDP-R, a control relationship is established, and an SLPI has been invoked.

3)	For a TDP-R, an SSF instruction is being prepared.

4)	For a TDP-N, no relationship is established. An SLPI has been invoked, executes and terminates.

SCF Postconditions (EDP):

–	For an EDP, the SCSM-FSM stays in the substate "Waiting for Notification or Request" if the message type was notification and there are still EDPs armed or a "CallInformationReport" or "ApplyChargingReport" requested; or

the SCSM-FSM moves to the state "Idle" if the message type was notification and there are no more EDPs armed, no "CallInformationReport" or "ApplyChargingReport" are requested; or

the SCSM-FSM moves to the state "Preparing SSF Instructions" if the message type was request.

##### 3.3.41.3.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP service which are used for reporting operation errors are described in 3.4.

### 3.3.42	Search procedureXE "Search procedure"§

#### 3.3.42.1	General description

The X.500 'Search' operation is used to search a portion of the SDF resident DIT for entries of interest and to return selected information from those entries. For a full description of the Search operation, see 10.2/X.511.

##### 3.3.42.1.1	Parameters

See 10.2.2/X.511 and 10.2.3/X.511.

#### 3.3.42.2	Invoking Entity (SCF)

##### 3.3.42.2.1	Normal procedure

SCF Precondition:

1)	SCSM: "SDF Bound" or "Wait for Subsequent Requests".

SCF Postcondition:

–	SCSM: "SDF Bound".

When the SCSM is in the state "Wait for Subsequent Requests" and a need of the service logic to search and/or read information of the SDF exists, an internal event [(e2) Request_to_SDF] occurs. Until the application process has not indicated with a delimitor that the operation should be sent, the SCSM remains in the state "Wait for Subsequent Requests" and the operation is not sent. The operation is sent to the SDF in a message containing a Bind argument. The SCSM waits for the response from the SDF. The reception of the response [(E5) Response_from_SDF_with_Bind or (E4) Bind_Error] to the Bind operation previously issued to the SDF causes a transition of the SCF to the state "SDF Bound" or to the state "Idle". When the SCSM has moved to state "Idle", the Search operation was discarded. In the State "SDF Bound", the response of the Search operation [(E7) Response_from_SDF] causes a transition of the SCF to the same state ("SDF Bound"). It may be either the result of the Search operation or an error.

When the SCSM is in the state "SDF Bound" and a need of the service logic to search and/or read information of the SDF exists, an internal event occurs. This event, called (e6) Request_to_SDF causes a transition to the same state "SDF Bound" and the SCSM waits for the response from the SDF. The reception of the response [(E7) Response_from_SDF] to the Search operation previously issued to the SDF causes a transition of the SCF to the same state "SDF Bound". The response from the SDF may be either the result of the Search operation or an error.

##### 3.3.42.2.2	Error handling

Generic error handling for the operation related errors is described in 10.2.4/X.511 and 10.2.5/X.511 and the TCAP services that are used for reporting operating errors are described in 2.2.2.

#### 3.3.42.3	Responding Entity (SDF)

##### 3.3.42.3.1	Normal procedure

SDF Precondition:

1)	SDSM: "SCF Bound"or "Bind Pending".

SDF Postcondition:

–	SDSM "SCF Bound".

When the SDF is in the state "Bind Pending", the external event (E3) Request_from_SCF caused by the reception of a 'Search' operation from the SCF occurs. The SDF does not proceed to the operation until a Bind operation has been successfully executed. It remains in the same state.

When the SDF is in the state "SCF Bound", the external event (E7) Request_from_SCF caused by the reception of a 'Search' operation from the SCF occurs. The SDF waits for the response to the operation.

On the receipt of the event (E7) and before retrieving the data as specified in the operation parameters, the SDF takes the following actions:

–	verify that the objects accessed by the request exists;

–	verify that the user on behalf of whom the request is performed has sufficient access rights to access the objects and attributes reached during the execution of the operation;

–	verify that attributes on which an operation should be performed exist in the object.

After the specified actions indicated above are successfully executed, the SDF returns all the possible attributes that satisfy the retrieval criteria to the SCF. The sending of the result corresponds to the event (e6) Response_to_SCF.

##### 3.3.42.3.2	Error handling

Generic error handling for the operation related errors is described in 10.2.4/X.511 and 10.2.5/X.511 and the TCAP services that are used for reporting operating errors are described in 2.2.2.

### 3.3.43	SelectFacility procedureXE "SelectFacility procedure"§

#### 3.3.43.1	General description

This operation is sent by the SCF to the SSF and requests the SSF to perform the terminating basic call processing actions to select the terminating line if it is idle, or selects an idle line from a multi-line hunt group, or selects an idle trunk from a trunk group, as appropriate. If no idle line or trunk is available, the SSF determines that the terminating facility is busy. Refer to 4.2.2.2/Q.1214 for additional call modelling related semantics.

##### 3.3.43.1.1	Parameters

–	alertingPattern:

See Recommendation Q.1290. It only applies if the network signalling supports this parameter or if SSF is the terminating local exchange for the subscriber.

–	destinationNumberRoutingAddress:

This parameter contains the called party number towards which the call is to be routed. The encoding of the parameter is defined in Recommendation Q.763.

–	iSDNAccessRelatedInformation:

This parameter contains (possibly multiple) information elements as per Recommendation Q.931. See Access Transport Parameter signalling information in Recommendations Q.762, Q.763 and Q.931. Refer to 6.4.4/Q.1214 (Analysed Information) for population rules for ISDN access related information.

–	calledFacilityGroup:

See Recommendation Q.1290.

–	calledFacilityGroupMember:

See Recommendation Q.1290.

–	originalCalledPartyID:

See Q.762 Original Called Number signalling information.

#### 3.3.43.2	Invoking Entity (SCF)

##### 3.3.43.2.1	Normal procedure

SCF Preconditions:

1)	A control relationship exists between the SCF and the SSF.

2)	An SLPI has determined that a “SelectFacility” is to be sent by the SCF.

SCF Postcondition:

–	SLPI execution may continue.

In the SCSM-FSM state “Preparing SSF Instructions”, this operation is invoked by an SCF if the service logic results in the request to an SSF to route a call to a specific destination and to continue call processing at the Select_Facility_and_Present_Call PIC. If no event monitoring has been requested and no reports (CallInformationReport and ApplyChargingReport) have been requested in a previously sent operation, an SCSM-FSM transition to the state “Idle” occurs. Otherwise, if event monitoring has been requested or any report (CallInformationReport and ApplyChargingReport) has been requested, the SCSM-FSM transitions to the state “Waiting for Notification or Request”.

##### 3.3.43.2.2	Error handling

If reject or error messages are received, then the SCSM informs the SLPI of the message and remains in the state “Preparing SSF Instructions”.

Generic error handling for the operation related errors is described in 3.2 and the TCAP service which are used for reporting operation errors are described in 3.4.

#### 3.3.43.3	Responding Entity (SSF)

##### 3.3.43.3.1	Normal procedure

SSF Preconditions:

1)	Call termination attempt has been initiated.

2)	Basic call processing has been suspended at a DP.

3)	The SSF waits for instructions.

SSF Postconditions:

1)	The SSF performs call processing action to route the call to the specified destination and applies the appropriate alerting pattern.

2)	In the T-BCSM, call processing resumes at the Select_Facility _and_Present_Call PIC.

##### 3.3.43.3.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP service which are used for reporting operation errors are described in 3.4.

### 3.3.44	SelectRoute procedureXE "SelectRoute procedure"§

#### 3.3.44.1	General description

This operation requests the SSF to perform the originating basic call processing actions to determine routing information and select routing for a call, based either on call information available at the SSF or on call information provided by the SCF. Based on the routing information provided, the SSF attempts to select a primary route for the call. If the route is busy, the SSF attempts to select an alternative route. The SSF may fail to select a route for the call if all routes are busy.

##### 3.3.44.1.1	Parameters

–	destinationRoutingAddress:

Represents a list of called party numbers (primary and alternates).

–	alertingPattern:

See Recommendation Q.1290. It only applies if the network signalling supports this parameter or if SSF is the terminating local exchange for the subscriber.

–	correlationID:

Used by the SCF to associate the 'AssistRequestInstructions' operation from the assisting SSF with the Request from the initiating SSF. The 'correlationID' is used in the context of a hand-off procedure and only if the correlation id is not embedded in the 'destinationRoutingAddress'.

–	iSDNAccessRelatedInformation:

Carries the same information as the protocol element ISUP Access Transport parameter in Recommendation Q.762.

–	originalCalledPartyID:

See Q.762 Original Called Number signalling information.

–	routeList:

See Recommendation Q.1290.

–	scfID:

Used by an assisting SSF to identify which SCF the 'destinationRoutingAddress' should be sent to.

–	travellingClassMark:

See Recommendation Q.1290.

–	carrier:

See Recommendation Q.1290. In this message, the carrier selection field is null (00000000) and Carrier ID indicates the carrier to use for the call.

#### 3.3.44.2	Invoking Entity (SCF)

##### 3.3.44.2.1	Normal procedure

SCF Preconditions:

1)	Call origination attempt has been initiated.

2)	Authority/ability to place outgoing call has been verified.

3)	Destination information is available in the SSF or provided by the SCF.

4)	Basic call processing has been suspended at one of the following DPs:

·	Origination_Attempt_Authorized;

·	Collected_Info;

·	Analysed_Info;

·	Route_Select_Failure;

·	O_Called_Party_Busy;

·	O_No_Answer;

·	O_Disconnect (called party disconnect only).

5)	A control relationship has been established and the SLPI is processing the incoming request.

SCF Postconditions:

1)	SLPI execution is terminated if no monitoring is requested.

2)	SLPI execution is suspended pending the monitored event occuring, if monitoring is requested.

The SelectRoute message requests the SSF-CCF to resume call origination processing taking into account the address and routing information provided in the message parameters. Call Processing resumes at the ROUTING & ALERTING PIC.

##### 3.3.44.2.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

#### 3.3.44.3	Responding Entity (SSF)

##### 3.3.44.3.1	Normal procedure

SSF Precondition:

–	A TDP or EDP request has been invoked.

SSF Postcondition:

–	The SSF performs the call processing actions to select the route.

The SSF-CCF shall resume call processing at the ROUTING & ALERTING PIC and proceed according to the BCSM description to attempt to complete the call over the designated route.

The following additional requirements also apply to the SelectRoute message:

·	Alternate route/carrier selection as indicated in the SelectRoute shall proceed until either an idle facility is found or until processing of the call encounters a screening feature which blocks the call. (Network operator specific.)

·	If a route requires a TCM, the value in the TCM parameter shall be used. If a TCM is required and the TCM parameter is not supplied in the SelectRoute, the SSF-CCF shall derive the TCM in accordance with requirements that apply assuming no IN involvement with the call.

SelectRoute and O_Called_Party_Busy, O_No_Answer:

·	When the SSF-CCF receives the SelectRoute message in response to an O_Called_Party_Busy message the SSF-CCF shall do the following:

1)	The SSF-CCF shall release any resources that were used to process the call between the ANALYSE_INFORMATION and ROUTING & ALERTING PICs.

2)	The SSF-CCF shall resume call processing at the ROUTING & ALERTING PIC, and process the message as described in this procedure description.

3)	If the originating access is DSS 1, then the SSF-CCF shall not send another CALL PROCeeding message (INAP-DSS 1 interworking is for further study).

·	When the SelectRoute message is received in response to an  O_No_Answer TDP-Request Message, the SSF-CCF shall do the following:

1)	The SSF-CCF, if it is providing audible ringing tone to the calling party, shall remove this tone.

2)	The SSF-CCF shall release any resources that were used to process the call between the ANALYSE_INFORMATION and ROUTING & ALERTING PICs.

3)	The SSF-CCF shall resume call processing at the ROUTING & ALERTING PIC, and process the message as described in this procedure description.

4)	If the originating access is DSS 1, then the SSF-CCF shall not send another CALL PROCeeding message (INAP-DSS 1 interworking is for further study).

·	When the SelectRoute message is received in response to an O_No_Answer EDP-Request message, the SSF-CCF shall do the following:

1)	The SSF-CCF, if it is providing audible ringing tone to the calling party, shall remove this tone.

2)	The SSF-CCF shall resume call processing at the ROUTING & ALERTING PIC, and process the message as described in this Recommendation.

3)	If the originating access is DSS 1, then the SSF-CCF shall not send another CALL PROCeeding message (INAP-DSS 1 interworking is for further study).

SelectRoute with OriginalCalledPartyID:

·	When the SSF-CCF receives a SelectRoute message that contains the OriginalCalledPartyID parameter, the SSF-CCF shall map the received OriginalCalledPartyID parameter to, for example (ISUP interworking is for further study):

1)	the OriginalCalledNumber information element in the Facility information element if the SSF-CCF does not yet have a value for the OriginalCalledNumber AND if the SSF-CCF routes the call to an ISDN line (for Basic rate and Primary rate ISDN signalling);

2)	the OriginalCalledNumber parameter in the IAM if the SSF-CCF does not already have a value for the OriginalCalledNumber AND if the SSF-CCF routes the call to an SS7 trunk (for ISDN-UP signalling).

##### 3.3.44.3.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

### 3.3.45	SendChargingInformation procedureXE "SendChargingInformation procedure"§

#### 3.3.45.1	General description

This operation is used to instruct the SSF on the charging information to be sent by the SSF. The sending of charging information can either be by charge pulses or signalling or internal if SSF is located in the Local Exchange (LE). In the LE, either charge meter can be updated or a standard call record created. A possibility exists for the SendChargingInformation (SCI) operation to be invoked on multiple occasions. The charging scenario supported by this operation is: 3.2 (refer to Appendix II/Q.1214, "Charging scenarios").

NOTE – The interworking between SSF and PSTN is network operator specific. This operation has much PSTN/IN interactions.

##### 3.3.45.1.1	Parameters

·	sCIBillingChargingCharacteristics:

This parameter indicates billing and/or charging characteristics. Its content is network operator specific. Depending on the applied charging scenario, the following information elements can be included (refer to Appendix II/Q.1214, "Charging scenarios"):

–	charge level (scenario 3.2);

–	chargePulses;

–	chargeMessages.

·	partyToCharge:

This parameter indicates where the charging information must be sent.

#### 3.3.45.2	Invoking Entity (SCF)

##### 3.3.45.2.1	Normal procedure

SCF Preconditions:

1)	A control relationship exists between the SCF and the SSF.

2)	An SLPI has determined that a "SendChargingInformation" has to be sent by the SCF.

SCF Postconditions:

1)	No FSM state transition;

2)	SLPI execution may continue.

The SCSM-FSM is in state "Preparing SSF Instruction" or is in state "Queueing FSM". The SendChargingInformation procedure shall be invoked by the SCF in accordance with the demands of the SLPI for relevant charging information. If appropriate this information shall be sent back down the call path.

This causes no SCSM-FSM state transition.

##### 3.3.45.2.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

#### 3.3.45.3	Responding Entity (SSF)

##### 3.3.45.3.1	Normal procedure

SSF Preconditions:

–	SSF-FSM: State c: "Waiting for Instructions"; or

SSF-FSM: State d: "Waiting for End of User Interaction"; or

SSF-FSM: State e: "Waiting for End of Temporary Connection"; or

SSF-FSM: State f: "Monitoring"; or

Assisting/hand-off SSF-FSM State b: "Waiting for Instructions".

SSF Postcondition:

–	No FSM state transition.

On receipt of this operation the SSF performs actions to send the charging information. The sending of charging information can either be by charge pulses or signalling or internal if SSF is located in LE. In the LE, either charge metre can be updated or a standard call record created. The interworking between SSF and PSTN is network operator specific. This operation has much PSTN/IN interactions.

For instance, by sending an operation "SendChargingInformation" the SCF instructs the SSF to initiate the PSTN/ISDN charging functions according to the given information about the charging level to use.

The charging level can be determined either by one of the following functions:

a)	the SCF; or

b)	the SSF; or

c)	the charging function in a succeeding exchange.

In case the SCF has determined the charging level, the "SendChargingInformation" operation contains the charging level to be applied.

In case the SSF determines the charging level, the "SendChargingInformation" operation contains the parameters to determine the charging level.

If the charging level was determined by the IN (SCF or SSF), the SSF provides the charging level to be applied to the PSTN/ISDN charging functions [cases a) and b)].

In case c, the charging level is determined in a succeeding exchange. The "SendChargingInformation" operation either contains the corresponding parameters indicating this fact or the SSF detects during trying to determine the charging level based on SCF provided parameters that the charging level shall be determined in a succeeding exchange. Based on already existing PSTN/ISDN capabilities, the SSF provides the PSTN/ISDN charging functions with the necessary information and backward charge messages shall be transferred down the call path when allowed by the SCF (generated by a succeeding exchange, for example an international gateway).

In the scenario described above, charging/billing is performed by means of existing mechanisms of the PSTN/ISDN initiated and controlled by the IN.

That means the determination of the charging method – on-line or off-line – and the items to be charged for shall be done in the basic network, just like the charge generation and the charge registration.

##### 3.3.45.3.2	Error handling

Generic error handling for the operation related errors is described in 3.2 and the TCAP services which are used for reporting operation errors are described in 3.4.

### 3.3.46	ServiceFilteringResponse procedureXE "ServiceFilteringResponse procedure"§

#### 3.3.46.1	General description

This operation is used to report the values of counters specified in a previous sent 'ActivateServiceFiltering' operation to the SCF.

##### 3.3.46.1.1	Parameters

–	countersValue:

The parameter contains the count of calls filtered during the filtering period. It is a list of counter identifications and the related values.

–	filteringCriteria:

This parameter is used to address the concerned service logic at the SCF.

–	responseCondition:

This parameter is used to identify the reason why the ServiceFilteringResponse is sent.

intermediateResponse indicates that service filtering is active, a call is received and the interval timer is expired; or

that service filtering is active and the threshold value, 'numberOfCalls', is reached.

lastResponse indicates that the duration time is expired and service filtering is stopped, or
that the stop time is met and service filtering is stopped.

#### 3.3.46.2	Invoking Entity (SSF)

##### 3.3.46.2.1	Normal procedure

SSF Preconditions:

1)	Service filtering is running and the interval time is expired and a call is received; or

2)	Service filtering is running and the threshold value is reached; or

3)	Service filtering has been finished (duration time expired or stop time met); or

4)	The operation 'ActivateServiceFiltering' is received and encounters an active service filtering entity.

SSF Postcondition:

–	Service filtering proceeds or is ended depending on the duration time.

The SSF sends the 'ServiceFilteringResponse' operation to the SCF. The 'filteringCriteria' parameter is provided to enable the addressing of the concerned service logic at the SCF.

Before 'ServiceFilteringResponse' is sent, it is checked whether call gapping criteria are met. If so, the 'ServiceFilteringResponse' is not sent and the counting continues without resetting the counters. The last 'ServiceFilteringResponse' (stop time is met or duration time expired) is sent without checking any call gap criteria.

After sending 'ServiceFilteringResponse' the service filtering counters are reset.

If service filtering proceeds after sending 'ServiceFilteringResponse' (e.g. interval time expired), the SSME-FSM remains in the state "Non_Call_Associated_Treatment".

If service filtering is stopped after sending 'ServiceFilteringResponse' (duration time expired or stop time is met), then the SSME-FSM moves to the "Idle Management" state. All allocated resources are released, i.e. the SSME-FSM is removed as well.

##### 3.3.46.2.2	Error handling

Operation related error handling is not applicable, due to class 4 operation.

#### 3.3.46.3	Responding Entity (SCF)

##### 3.3.46.3.1	Normal procedure

SCF Preconditions:

1)	Service filtering is running.

2)	The SCME is in the state "Waiting for Service Filtering Response".

SCF Postcondition:

–	The SCME forwards the received counter values to the SLPI.

The operation is handled by the Service Filtering FSM part of the SCF Management Entity (SCME). The SCME passes the received counter values to the SLPI where they are added to previously received counter values.

The "filteringCriteria" parameter as provided in "ServiceFilteringResponse" is used to address the SCME and the concerned service logic instance.

The Service Filtering FSM of the SCME remains in the state "Waiting For SSF Service Filtering Response" until the internal service filtering duration time in the SLPI expires. Then the SLPI informs the SCME about timer expiration. Now the SCME moves to the state "Service Filtering Idle".

##### 3.3.46.3.2	Error handling

Operation related error handling is not applicable, due to class 4 operation.

### 3.3.47	SpecializedResourceReport procedureXE "SpecializedResourceReport procedure"§

#### 3.3.47.1	General description

This operation is used as the response to a "PlayAnnouncement" operation when the announcement completed indication is set.

##### 3.3.47.1.1	Parameters

–	None.

#### 3.3.47.2	Invoking Entity (SRF)

##### 3.3.47.2.1	Normal procedure

SRF Preconditions:

1)	The SRSM-FSM is in the state "User Interaction".

2)	A "PlayAnnouncement" operation is being executed for which the parameter "RequestAnnouncementCompelete" was set TRUE.

3)	All information has been sent to the user.

SRF Postconditions:

1)	The SRSM-FSM remains in the same state.

2)	If the "DisconnectFromIPForbidden" parameter was set FALSE, the SRSM initiates a bearer channel disconnect sequence to the SSF using the applicable bearer channel signalling system after sending the "SpecializedResourceReport" operation to the SCF. The SRSM-FSM moves to the state "Idle".

##### 3.3.47.2.2	Error handling

Operation related error handling is not applicable, due to class 4 operation.

#### 3.3.47.3	Responding Entity (SCF)

##### 3.3.47.3.1	Normal procedure

SCF Precondition:

–	The SCSM-FSM is in the state "User Interaction", substate "Waiting for response from the SRF".

SCF Postconditions:

1)	The SCSM-FSM remains in the same state.

2)	If the "SpecializedResourceReport" relates to a "PlayAnnouncement" operation with permission of SRF initiated disconnection, the SCSM-FSM moves to the state "Preparing SSF Instructions".

##### 3.3.47.3.2	Error handling

Operation related error handling is not applicable, due to class 4 operation.