|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	R.30

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TELEGRAPHY

TELEGRAPH  TRANSMISSION

TRANSMISSION  CHARACTERISTIC  FOR  INTERNATIONAL  VFT  LINKS

ITU-T  Recommendation  R.30

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation R.30 was published in Fascicle VII.1 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation R.30

Fascicle VII.1 - Rec. R.30

TRANSMISSION CHARACTERISTIC FOR INTERNATIONAL VFT LINKS

(Mar del Plata, 1968; amended at Geneva, 1976)

1	Standardized carrier systems with 4-kHz and 3-kHz spacing permit homogeneous voice-frequency telegraph (VFT) systems providing the capacities of telegraph channels given in Table 1/R.30.

TABLE 1/R.30

| Bearer bandwidth | 50-baud 120-Hz spacing | 100-baud 240-Hz spacing | 200-baud 360-Hz spacing | 200-baud 480-Hz spacing |
| --- | --- | --- | --- | --- |
| 4 kHz | 24 | 12 | 8 (not normally used) | 6 |
| 3 kHz | 22 | 11 | 7 | 5 |

2	Audio-frequency circuits with heavy or semi-heavy loading permit 12-channel 50-baud systems; circuits with lighter loading permit 18 channels at 50 bauds.

3	Four-wire links are to be preferred for voice-frequency telegraphy.

4	The composition of a 4-wire link for voice-frequency telegraphy differs from that of a telephone circuit in that there are no terminating sets, signalling equipment and echo suppressors.

5	With 2-wire links, a duplex arrangement would not be feasible since the links could not be balanced with the necessary precision to avoid mutual interaction. If the low frequencies are used for transmission in one direction and high frequencies for the other direction, a 2-wire link can be used for voice-frequency telegraphy.

6	The conditions of use of international VFT links are described in detail in Recommendation H.22 [1].

7	PCM (pulse code modulation) channels complying with Recommendation G.712 [2] are also suitable as bearers for FMVFT (frequency-modulated voice-frequency telegraph) links. However, the increase in telegraph distortion in relation to the transmission level and the number of tandem-connected PCM channels is the subject of further study.

References

[1]	CCITT Recommendation Transmission requirements of international voice-frequency telegraph links (at 50, 100 and 200 bauds), Rec. H.22.

[2]	CCITT Recommendation Performance characteristics of PCM channels at audio frequencies, Rec. G.712.